# New Key Support for Microsoftâ Windowsâ Operating Systems and Applications

# (Optional Specification)

Specification :	0.0

Filename:	NEWKEYS.DOC

Date:		July 30, 2026

# Introduction

In the effort to continue extending and refining the accessibility of the Windows graphical user interface Microsoft will be enabling three new keys on new keyboard designs in the Windows 95 time frame.  The three new keys are:

·	Application key

·	Right Windows key

·	Left Windows key

The keys will have new assigned scan codes (see appendix).  Many applications shipping after/around mid-95 should support the new codes.  This document describes what the keys do and how the applications should use them.

# New keys

During the development of Windows 95 and our own hardware peripheral projects, corporate customer and end user feedback convinced us that additional refinements to the standard 101 key keyboard would allow us to offer more useful functionality in the Windows operating system to the keyboard user..  These three new keys simplify existing keystrokes like Alt-Tab, Ctrl-Esc or right mouse button clicks to single key operations.  Extensive testing of the Windows user interface showed us that a very small percentage of Windows users are aware of these key combinations or their use. These optional three new keys provide a consistent mechanism for the end user to access functionality within the Windows operating system and within individual applications

## Below is the listing of actions each key should cause.  Note that the actions are not affected by the NumLock key state.

## Application key

When the user presses the unmodified Application key, the application brings up the context (pop-up) menu at the current select position.  This is equivalent to pressing the right mouse button in some applications today.  Pressing of the Application key will not disturb the current mouse pointer position.  See the appendix for the Application key scan codes.

# Recommeded ISVs Support

Applications key = Context Menu key (similar to right mouse button click in some current applications)

Other application specific functionality is allowed but the key should be configurable by the user to set its functionality in these cases.

µ §

Figure 1 -- Context Menu on Caret Position

## Alt+Application key

Unassigned, TBD by application.  For example in some Microsoft applications Alt+Application key brings up the Properties dialog on the current caret position (selected object).  See Figure 1.

## Shift+Application key

Unassigned.  TBD by application. Examples of functionality can be having access to an automated help agent, setting focus to a toolbar, or performing an automated macro task.

## Ctrl+Application key

Unassigned.  TBD by application.

## Application key + any other key (Except Windows Key)

Use of the Application key as a modifier key must be supported within the application but can be used to provide even broader application support of the keyboard.  There will not be Microsoft operating system support for the Applications key to be used as a modifier key.

## Standard 101 keyboard compatibility

We strongly suggest that new functionality be provided when the user uses ‘old’ style 101 key keyboard.  The following is a suggested mapping of the new key functionality when old keyboard is used.

## Application key

Shift+F10 should be used for bringing up the context menu, for instance.

## Alt+Application key

Alt+Shift+F10 should be used for invoking properties dialog, for instance.

## Shift+Application key

Not applicable.

## Ctrl+Application key

Not applicable.

## Left Windows key

Will set focus to the Windows 95 User Interface.  Note that the application does not have to do any work to support the Windows keys.

## Right Windows key

Same functionality as left Windows key (different scan code, however). Note that the application does not have to do any work to support the Windows keys.

## Left+Right Windows key

Log-on keys for Windows NTä logon/logoff. Note that the application does not have to do any work to support the Windows keys.

## Windows key +any key

Both the left and right Windows key can be used as a modifier key in Windows 95.  All Win+ keys are reserved for system hot keys and should NOT be used by applications (eg:  Win+F could be short cut for Find File).

Note: The Windows keys functionality are owned by the operating system.  Only shell applications should implement these keys and then they should preserve and extend on the functionality that will be established in the Windows 95 shell.  This consistency of operation is the critical value that these new Windows keys provide to the end user.

# Appendix A:  New scan codes

The virtual key codes described in the following tables are specifically for Windows operation only and are not generated by the keyboard directly.

For the three  new scan code values, we will use the following:

For scan code set 1:

| New Key | Scan Code Set 1: Base Case, or Shift+Num Lock Scan Code (Make) | Scan Code (Break) | Windows Virtual Key |
| --- | --- | --- | --- |
| Left Window Key | 0xE0 0x5B | 0xE0 0xDB | 5B |
| Right Window Key | 0xE0 0x5C | 0xE0 0xDC | 5C |
| Applications Key | 0xE0 0x5D | 0xE0 0xDD | 5D |

| New Key | Scan Code Set 1: Shift Case Scan Code (Make) | Scan Code (Break) | Windows Virtual Key |
| --- | --- | --- | --- |
| Left Window Key | 0xE0 0xAA 0xE0 0x5B | 0xE0 0xDB 0xE0 0x2A | 5B |
| Right Window Key | 0xE0 0xAA 0xE0 0x5C | 0xE0 0xDC 0xE0 0x2A | 5C |
| Applications Key | 0xE0 0xAA 0xE0 0x5D | 0xE0 0xDD 0xE0 0x2A | 5D |

| New Key | Scan Code Set 1: Numlock ON Scan Code (Make) | Scan Code (Break) | Windows Virtual Key |
| --- | --- | --- | --- |
| Left Window Key | 0xE0 0x2A 0xE0 0x5B | 0xE0 0xDB 0xE0 0xAA | 5B |
| Right Window Key | 0xE0 0x2A 0xE0 0x5C | 0xE0 0xDC 0xE0 0xAA | 5C |
| Applications Key | 0xE0 0x2A 0xE0 0x5D | 0xE0 0xDD 0xE0 0xAA | 5D |

For scan code set 2:

| New Key | Scan Code Set 2: Base Case, or Shift+Num Lock Scan Code (Make) | Scan Code (Break) | Windows Virtual Key |
| --- | --- | --- | --- |
| Left Window Key | 0xE0 0x1F | 0xE0 0xF0 0x1F | 5B |
| Right Window Key | 0xE0 0x27 | 0xE0 0xF0 0x27 | 5C |
| Applications Key | 0xE0 0x2F | 0xE0 0xF0 0x2F | 5D |

| New Key | Scan Code Set 2: Shift Case Scan Code (Make) | Scan Code (Break) | Windows Virtual Key |
| --- | --- | --- | --- |
| Left Window Key | 0xE0 0xF0 0x12 0x1F | 0xE0 0xF0 0x1F 0xE0 0x12 | 5B |
| Right Window Key | 0xE0 0xF0 0x12 0x27 | 0xE0 0xF0 0x27 0xE0 0x12 | 5C |
| Applications Key | 0xE0 0xF0 0x12 0x2F | 0xE0 0xF0 0x2F 0xE0 0x12 | 5D |

| New Key | Scan Code Set 2: Numlock ON Scan Code (Make) | Scan Code (Break) | Windows Virtual Key |
| --- | --- | --- | --- |
| Left Window Key | 0xE0 0x12 0xE0 0x1F | 0xE0 0xF0 0x1F 0xE0 0xF0 0x12 | 5B |
| Right Window Key | 0xE0 0x12 0xE0 0x27 | 0xE0 0xF0 0x27 0xE0 0xF0 0x12 | 5C |
| Applications Key | 0xE0 0x12 0xE0 0x2F | 0xE0 0xF0 0x2F 0xE0 0xF0 0x12 | 5D |

For scan code set 3:

| New Key | Scan Code Set 3: Scan Code (Make) | Scan Code (Break) | Windows Virtual Key |
| --- | --- | --- | --- |
| Left Window Key | 0x8B | 0xF0 0x8B | n/a |
| Right Window Key | 0x8C | 0xF0 0x8C | n/a |
| Applications Key | 0x8D | 0xF0 0x8D | n/a |

### LWIN, RWIN, and APPS Keys as modifiers

The 104 key keyboard should generate unique scan codes for make and break.  Any of the three new keys may be treated as modifier keys, much in the same way as Alt, and Control keys are handled today.  It will be up to the operating system to decide on an implementation scheme for the modifier key functionality.

# Recommended Physical Key Locations

The opportunity exists on 101/2 key keyboard designs to add the three new keys to the bottom row adjacent to the space bar. The Windows keys and the applications keys can be used as modifier keys so they should occupy space near the existing Ctrl, Shift and Alt positions.  Location of the new keys is up to the keyboard designer and is anticipated to vary between laptop and desktop configurations.

# Licensing the Windows and Applications Key Logos

Keyboard vendors, or OEMs and software ISVs can get the logo licensing requirements to allow them to use the key logos on their keyboards or in their documentation.  The Windows Logo is a valuable Microsoft trademark. Licensing the logos is free but requires a license agreement for use on keyboards.  For use in documentation, while no license agreement is required, Microsoft’s trademark guidelines must be observed.  These guidelines, and information on licensing the Windows and Applications Key Logos are available from Microsoft. Send a fax to the Microsoft Windows Key Logo, Microsoft Systems Marketing at (206) 936 7329 or email logo@microsoft.com on the Internet to get more information about the program.

* The logo licensing information, guidelines and artwork will be available after September 6, 1994.

Microsoft is a registered trademark and MS-DOS, Windows NT and Windows are trademarks of Microsoft Corporation.