DEC 1994

NetBIOS Emulator Installation

for

D-Link PCMCIA Ethernet Caed

=================================

The D-Link NetBIOS Emulator is a standardized network interface program that

allows this Adapter to run NetBIOS-based network operating systems like the IBM

PC LAN Program and others.

No software modification or additional installation is necessary.

All operations as well as installation procedures for the network operating

system will remain unaltered.

The following files will be used in this installation:

Files		  Contents

-----------	  ------------------------------------

DE-65N.EXE        DE-650 Driver program.

NBIOS.EXE	  NetBIOS Emulator Program.

Note:

Before install NetBIOS driver, run PCMCIA initial program on

A:\IBMPC\INSTALL directory.

Please see as following:

*** PC installed Socket and Card services ***

Skip this step

OR

*** PC installed Socket service or Databook Cardtalk***

*** PC's PCMCIA controller is Intel 82365SL ***

*** PC is Toshiba T3300SL ***

DCD16.EXE [/iop:P0] [/irq:P1] [/slt:P2]

Please refer to \IBMPC\INSTALL\README.DOC for detail information.

2. Parameters for INSTALL utility:

DCD16.EXE

DCD16.EXE is an integration verrsion of open adapter utility.

DCD16.EXE supports Intel 82365SL, Toshiba T3300SL's ICCNT, and Socket

Services v1.01, v2.0, v2.01, v2.10 Card Services v2.0, v2.01, v2.10.

For more detail information, please refer to \IBMPC\INSTALL\README.DOC.

Example:

device=DCD16.EXE [/iop:P0] [/irq:P1] [/slt:P2]

Parameter description:

/iop   :I/O base

/irq   :IRQ value

/slt   :Slot number

P0 :The selectable window I/O base with 0x20 range.

The default value is 0x300.

P1 :The selectable irq value.

The default value is 0x5.

P3 :The selectable slot number.

The default value is 0. You must set 0 when you use slot 1.

Set slt:1 when slot 2 is used,etc.

To install the NetBIOS Emulator, do as follows:

1. Load in the driver program DE-65N.EXE.

>DE-65N

2. Load in the NetBIOS Emulator program NBIOS.EXE.  This program is

memory-resident and will stay in your computer's memory until its power is

turned off or until you re-boot.

>NBIOS [/Sm] [/Ck]

Typing the parameter after NBIOS is optional. The parameter is used as

follows:

/Sm   Specifies the maximum number of sessions allowed on your

computer, where m can be a value from 1 to 254.  If this

parameter is not specified, the default number of sessions

will be set to 32.

/Ck   Specifies the maximum number of NetBIOS command available for

your computer where k can be a value from 1 to 255.	If this

parameter is not specified, the default number of NetBIOS

commands will be set to 32.

Your computer now is ready to run a NetBIOS-based network operating system.