������ ������ �������������� ������� ����������������   Ver. 3.0

������ ��     ������   ��    �����   �� �� ��   ��

�� �� ��� ��  ��   ��    ��      ��    ��   ��

������ ������ ��  ��   ��    ��      ��    ��   ��  (tm)

(C)opyright 1993, Seagate Technology, Inc.

Scotts Valley, California USA

Tech Support BBS (408)438-8771

Introduction (see License agreement at the end of this document)

----------------------------------------------------------------

!  READ THIS ENTIRE DOCUMENT BEFORE USING THIS PROGRAM.  THIS PROGRAM

!  IS DESTRUCTIVE TO USER DATA.  SEVERAL SPECIFIC WARNINGS AND

!  RECOMMENDATIONS ARE GIVEN THAT MAY PERTAIN TO YOUR DISC DRIVE.

SGATFMT3 (Seagate Format) is a lo-level formatting utility designed

for AT 286/386/486 systems, only.  (If the program is run on an XT,

most likely a stack overflow error message will display.)

SGATFMT3 does not use the system BIOS to access the drive, but instead

uses the AT register command set.  This means that it is not necessary

to pre-set a CMOS drive-type prior to the lo-level format.  The CMOS

drive type will become mandatory, however, prior to partitioning and

the DOS hi-level format (see the section below on SETTING CMOS DRIVE

TYPES).

SGATFMT3 only works if the controller/host adapter is set to the

primary hard drive port addresses of 1F0-1F7. (This is the common

port address used on most controllers.)

SGATFMT3 checks to see if a Seagate ST21/22 M or R controller is

installed with its on-board controller bios enabled.  If this

condition exists, SGATFMT3 will exit and issue an appropriate debug

command to initiate the controller's built in lo-level format.

SGATFMT3, in this v3.0 release, is designed and LIMITED to work with

the following Seagate disc drive interfaces: ST412 (both MFM and RLL),

ESDI (with controller bios disabled), and AT/IDE (with certain

limitations).  SCSI interface disc drives are not supported.  (See the

section "ABOUT DRIVES NOT LISTED")

==========================================================

There are three basic steps to preparing a hard disc drive for use in

a computer system:

1. Lo-level format (MFM, RLL, ESDI, and some SCSI)

2. Partitioning with the operating system software.

3. Hi-level formatting with the operating system software.

SGATFMT3 addresses step number 1.

==========================================================

The opening first screen is used to determine which of two drives is

to be selected for the lo-level format.  If you only have one drive

then select drive 0 by pressing 0, followed by the Enter key:

������ ������ �������������� ������� ����������������   Ver. 3.0

������ ��     ������   ��    �����   �� �� ��   ��

�� �� ��� ��  ��   ��    ��      ��    ��   ��

������ ������ ��  ��   ��    ��      ��    ��   ��  (tm)

���������������������������������Ŀ

� ������������������������������� �

� ������������������������������� �

� ������������������������������� �

�����������������������������������������

�                                       �

�������������������������������>  Drive 0                �

�                �    �                                  �

�                �                                       �

�                �����������������������������������������

�

�������������< (Look for your choice to show up here)

Please select physical hard drive 0 or 1 press <ret> to select

After the drive selection is made, the next step is to identify the

model:

���������������������������������Ŀ

� ������������������������������� �

� ������������������������������� �

� ������������������������������� �

�����������������������������������������

�                                       �

�                Drive 0                �

�    �   ������>  ST124                 �

�        �                              �

��������߳�������������������������������

�

���< (Look for your choice here)

Please select a Seagate drive model, press <ret> to select

U =prev D =next  HOME =first  END =last  PGUP =U10  PGDN =D10

Once the model has been identified and the Enter key is pressed , the

Main Menu appears:

������ FORMAT OPTIONS ������ͻ

�                            �

�  1. Format Drive           �

* �  2. Enter Defects          �

�  3. Verify Drive           �

�  4. Format/Verify Drive    �

�  5. Choose Another Drive   �

* �  6. Optimize Interleave    �

�                            �

����������������������������ͼ

(* Menu option not available when AT/IDE ZBR drive selected.)

1. Format Drive :  This is the "meat and potatoes" part of

SGATFMT3.  When this selection is made a warning appears, letting you

know that ALL DATA WILL BE ERASED.  This is very serious business!  If

you haven't backed up your data, then STOP!  Under no circumstances is

Seagate responsible for lost data.  If you elect to go on, you will be

asked to select or test for the proper interleave value. Next you will

be queried for head and cylinder skew values (see INTERLEAVE and

SKEWING sections below).  A format on a disc drive is very controller

dependent and usually means that the format performed by one

controller cannot be utilized by another.

2. Enter Defects :  Affixed to the top of every Seagate MFM

and RLL disc drive, is a list of micro-defects that were found to

exist at the time of manufacture.  Seagate's original list should

contain less than 1 defect per formatted megabyte and defect-free on

the first two cylinders.  The micro-defects that have been detected

are generally of two types: hard and soft.  A hard defect is usually a

surface problem and a soft defect is usually a magnetic anomaly of

some kind.  Soft defects are discovered at the factory with very

sophisticated test equipment, while hard defects can be discovered

with conventional software like SGATFMT3.

The typical defect label on the top of the drive is usually made up of

three columns: Cyl  Hd  BFI     and might look like this:

67   0  7814

68   0  7815

69   0  7816

175   2  3316   and so on.

The column heading "BFI" stands for Bytes From Index.  It may also be

listed as "BCAI" which stands for Byte Count After Index, and is the

same thing.  The Index pulse is usually generated by a Hall sensor

that is imbedded in the spindle motor or else it is encoded on servo

tracks.  This index pulse is considered the absolute point of

reference for the BFI or BCAI count.  With BFI, an individual sector

can be located and locked out as opposed to locking out the entire

track.  If a defect is entered in SGATFMT3 without a BFI (a BFI of 0),

then the entire track is locked out . Once all of the defects are

entered, the specific areas will be marked as bad upon exiting the

module. (see ANATOMY OF A SECTOR below)

3. Verify Drive :  This module should still be proceeded by a

complete backup before use.  Verify is available to search out hard

defects.  If the micro-defect list has been removed from the drive or

the suspicion of a new defect arises, then Verify can be run.  It will

report to the screen, and optionally to the printer, a cylinder, head,

and sector reference. Unfortunately, a specific BFI cannot be

reported. Therefore, if a subsequent lo-level format is performed, a

BFI of 0 will need to be entered.  Verify will ask if you want to do

destructive pattern testing.  If answered "No", the program operates

in a read-only mode.  If answered "Yes", you can choose up to nine

different patterns that are used in write-read mode.  (Note: a high

capacity drive may take several hours to complete if all nine patterns

are selected.)

4. Format/Verify Drive :  This function combines the Format

and Verify procedures into a single operation.  This step does

provide, however, for marking out "discovered" defects at the sector

level instead of whole tracks at the time of formatting.

5. Choose Another Drive : If two physical drives are

installed, this allows for switching between them.  Be ABSOLUTELY SURE

you are aware of which drive is selected.  The next saddest person in

the world is the one who formats the wrong drive! (Chin up.. worse

things can happen.)

6. Optimize INTERLEAVE : The interleave value for a hard disc

drive determines how many times a disc needs to spin in order to read

a single track of data.  The typical disc drive usually spins at 3,600

rpm (or 60 times per second).  On a MFM disc drive with 17 sectors per

track, the Read/Write heads, drive circuitry, controller and CPU are

required to process all 17 sectors in 1/60th of a second.  SGATFMT3

can test the system and report which interleave yields the fastest

data transfer rate for your system (this is a data destructive test,

be sure to back up 100% of your data before running the interleave

tests).  The best interleave possible is 1 to 1, meaning 1 revolution

to read 1 track of data. Interleaves are always whole numbers, so the

next best interleave is 2 to 1.

1 to 1:

1- 2- 3- 4- 5- 6- 7- 8- 9-10-11-12-13-14-15-16-17

(with sector 17 looping around to meet sector 1)

2 to 1:

1-10- 2-11- 3-12- 4-13- 5-14- 6-15- 7-16- 8-17- 9

(with sector 9 looping around to meet sector 1)

It takes a little getting used to looking at this, but the most

important fact to keep in mind is that the operating system reads the

sectors in sequential order and will read on until the next sector in

sequence appears.  On the 2 to 1 interleave example the disc will need

to spin two times in order to read all 17 sectors.  Most of today's

modern controllers are designed for a 1 to 1 interleave.  Some early

16-bit controllers for 286's were only 3 to 1 or 2 to 1.

An interesting problem happens if a 1 to 1 interleave is selected on a

controller not designed for this speed:  The Disc ends up performing

like it has a 17 to 1 interleave!  The reason for this is quite

simple.  If sector 2 immediately follows sector 1, and the controller

isn't ready to read sector 2, then the disc needs to spin all the way

around again in order to pick up on sector 2.  This extra spin would

be needed for all 17 of the sectors.

==========================================================

SKEWING

--------

By way of an analogy, the function of the modern disc drive has been

described like this: "Today's new generation of disc drives achieve

the engineering equivalent of a Boeing 747 flying at MACH 4 just two

meters above the ground, counting each blade of grass as it flies

over.  The read/write head floats at 12 millionths of an inch above

the surface of the disc which is turning at 3,600 revolutions per

minute. Read/write heads position precisely over information tracks

which are 800 millionths of an inch apart and the data is

electronically recorded at 20,000 bits per inch."

Skewing is best understood by first looking at the layout of a

non-skewed disc drive.  With the limitations of a two-dimensional

drawing, a single circular MFM track has 17 sectors and would look

like this:

1- 2- 3- 4- 5- 6- 7- 8- 9-10-11-12-13-14-15-16-17

(with sector 17 looping around to meet sector 1)

The platters within the drive are spinning at a very high rate

(usually 3,600 rpm), so one sector is passing beneath the R/W head

once every 980 millionths of a second!  This is obviously a very small

timing window.  When the entire track is processed, it is time to move

to the next head (on another surface) in the cylinder.  For example: a

drive with two heads reads track 1 head 1, track 1 head 2, then

repositions the heads over the next track and reads track2 head 1,

track 2 head 2, and so on.  The time it takes to switch between heads

is extremely fast since it is an electronic change.  The time it takes

to reposition over another cylinder, however, takes significantly

longer since it requires a mechanical movement that is an order of

magnitude slower.

Looking again at the 17 sectors, if we stack two heads we see:

head 1 :  1- 2- 3- 4- 5- 6- 7- ...... -17

head 2 :  1- 2- 3- 4- 5- 6- 7- ...... -17

We would expect sector 1 on head 2 to immediately follow sector 17 on

head 1.  Unfortunately, this doesn't happen because it TAKES TIME (or

"overhead") to switch to the new head, and by the time it does, sector

1 has already gone by! Therefore the R/W head waits for the disc to

spin around once for sector 1 to show up again so it can get on with

its job.  Effectively, we have wasted one disc revolution that equals

1/60th of a second which could have processed almost an entire track

of 17 sectors.  This is the crux of the problem that skewing

addresses: eliminating unnecessary disc revolutions.

The solution is easy; shift the beginning position of sector 1 head 2

enough to compensate for the head switching overhead.  That way when

head 1 sector 17 finishes and the head switches, sector 1 head 2 would

be spinning into place.  Remembering that tracks are circular, it

would look like this:

head 1 :  1- 2- 3- 4- 5- 6- 7- ...... -15-16-17

head 2 : 16-17- 1- 2- 3- 4- 5- ...... -13-14-15

\--|--/

|

Shifting these two sectors gives us time to allow for

the head switching overhead and is the equivalent to HEAD SKEW = 2.

In normal use, a disc drive switches heads many times more often than

it does switching physical cylinders.  The data throughput can rise

dramatically when a head skew is in place.  For example, a simple

non-head skewed MFM drive might have a transfer rate of 380kps and the

transfer rate of a drive with a head skew of 2 could rise to around

425kps. (Since we've listed a kind of performance result, here, it is

VERY important to point out that ALL systems/controllers have

different amounts of overhead and processing power, not to mention the

wide range of results from different transfer rate diagnostics. See

the section ABOUT TRANSFER RATES below.)

A formula for calculating a head skew value is as follows (but be sure

to read on):

HEAD SKEW =

[( head switch time * SPT * spindle speed ) / 60,000 ] + 2

Ex:     [( <15 �S * 17 * 3600 ) / 60,000 ] + 2 = 2

��������������������������������

�

Basically, this evaluates to zero, and the 2 is a typical overhead for

most MFM controllers.

Cylinder skewing is usually a little more drastic.  It stands to

reason that since the mechanics of repositioning the head assembly is

going to be significantly slower than an electronic head switch, the

value for a cylinder skew will be larger.  Going back to our two head

drive, we might see:

Cyl 1:  head 1 :  1- 2- 3- 4- 5- 6- 7- 8- 9-10-11-12-13-14-15-16-17

head 2 : 16-17- 1- 2- 3- 4- 5- 6- 7- 8- 9-10-11-12-13-14-15

Cyl 2:  head 1 :  8- 9-10-11-12-13-14-15-16-17- 1- 2- 3- 4- 5- 6- 7

\-----------|-----------/

|

Shifting these eight sectors gives us

time to account for the cylinder switching overhead and is the

equivalent to CYLINDER SKEW = 8.

A formula for calculating a cylinder skew value is as follows:

CYLINDER SKEW =

[( max track to track time * SPT * spindle speed ) / 60,000 ] + OHFactor

Ex:     [( 8 msec * 17 * 3600 ) / 60,000 ] + 0 = 8 (ok to round

down on MFM)

Note: OHFactor is an 'overhead factor' that is tied to SPT or sectors

per track. After some casual experimentation, we've figured -

SPT             OHFactor

17              0 or 1     (usually MFM drives)

26 - 31         1 or 2     (usually RLL drives)

33 - 52         2 or 3     (usually ESDI drives)

53 - >>         3 or 4     (usually high end ESDI drives)

The "0 or 1" type values are intended to be ambiguous, and are meant

to illustrate that these values are system/controller dependant. The

higher of the two numbers is the most conservative.  Generally,

choosing a value a little high is not as bad as choosing a value too

low, thereby causing a wasted disc revolution.  Now is a good time to

recall that it is the head skew value that offers the most significant

boost to the transfer rate, while an optimized cylinder skewing helps

only when the heads are repositioned over a different track.  If you

use a transfer rate utility to measure performance results, be advised

that many of them just use a single cylinder and don't reflect

cylinder skews.

==========================================================

ABOUT DRIVES NOT LISTED

-----------------------

Some points about lo-level formatting drives not listed above:

In the case of all SCSI drives:

These drives use a controller (properly called a host adapter) that

has an onboard BIOS chip. Coded within this bios chip is a lo-level

format utility (called 'firmware' as opposed to 'software') which can

initiate special SCSI commands. The fact that virtually all SCSI host

adapters have this capability, precludes the need for a stand-alone

software utility like SGATFMT3. Defect management on SCSI drives is

handled at the factory and/or by the drive "on-the-fly" on more

advanced drives, and is transparent to the user.  Access to the SCSI

host adapter's lo-level format utility is usually through the DOS'S

DEBUG utility.  Typically, you would start DEBUG, and then at the

"hyphen prompt" (DEBUG's user-friendly interface), type "G=C800:5"

without quotes and followed by ENTER (where C800 is the BIOS upper

memory address selected by jumpers on the host adapter).

In the case of ESDI drives:

These drives normally use a controller with an onboard BIOS that has

the lo-level utility.  Many ESDI drives have cylinder counts that

exceed the DOS limitation of 1024.  The ESDI controller's on-board

bios is required to "translate" these values in order to achieve full

capacity from the drive.  Defect management for ESDI drives has been

simplified over that of typical MFM drives. The manufacturer has

placed a small file on the drive which lists the coordinates of the

defects (cylinder, head, and BFI or BCAI) that can be read by the

controller, thereby eliminating the need to enter them by hand. Access

to the ESDI controller's lo-level format utility is usually through

the DOS'S DEBUG utility.  Typically, you would start DEBUG, and then

at the "hyphen prompt" (DEBUG's user-friendly interface), type a GO

command, -G=C800:5 (where C800 is the BIOS upper memory address

selected by jumpers on the controller). ESDI drives can be defined

optionally, with the BIOS on the controller card disabled, in a

user-definable or custom CMOS drivetype.  SGATFMT3 supports this

bios-disabled condition.

In the case of RLL drives :

These drives also normally use a controller like the ST21/22R

controllers with an onboard BIOS that has the lo-level utility. Defect

management for RLL drives is the same as MFM drives.  Defects are

usually listed on a sticker affixed to the top of the drive and need

to be entered manually during the lo-level format.  Access to the RLL

controller's lo-level format utility is usually through the DOS'S

DEBUG utility.  Typically, you would start DEBUG, and then at the

"hyphen prompt" (DEBUG's user-friendly interface), type a GO command,

-G=C800:5 (where C800 is the BIOS upper memory address selected by

jumpers on the controller). RLL drives can be defined optionally, with

the BIOS on the controller card disabled, in a user-definable or

custom CMOS drivetype.  This version of SGATFMT3 supports RLL drives

that are fully defined in CMOS with the controller BIOS disabled.

In the case of AT (IDE) drives:

AT (IDE) drives can be divided into three separate scenarios: Early,

Swift and ZBR.

1.  EARLY:  When AT interface drives (aka IDE - integrated drive

electronics, but so are SCSI's) were first introduced (ST157A family),

we strongly warned and cautioned against any attempt to lo-level

format the drives because 1) the factory written defect-mapping files

might be erased on reserved areas of the drive, and 2) the optimized

interleave and skewing values used would be forfeited giving slow

transfer rates.  At this stage of development, SGATFMT3 lists these

drives only as a fall back option, in lieu of a factory repair format.

If the drive has somehow lost its original format, or the partition

structure been corrupted by a virus etc., SGATFMT3 could be used to

reformat _without_ the benefit of the defect mapping files.  Any

defects will need to be "rediscovered" again; first, by the DOS high

level format and second, by a third-party disk scanning utility.

These utilities are quite likely to locate all of the hard errors, but

unlikely to find the soft errors.  The only way to completely evaluate

a drive for both hard and soft error is by a factory repair with

extremely sophisticated diagnostic equipment.  (See the glossary

section for HARD and SOFT ERRORS.)

2.  SWIFT:  As the AT interface products became more sophisticated

with new technology and the introduction of the Swift drives (models

like ST1239A, ST1201A etc), lo-level formatting became pretty much

"half" of a problem.  When these drives are in translation mode

(non-physical geometry definitions), a lo-level format is harmless to

the factory defect-mapping files and optimized skewing (albeit

destructive to user data) since it doesn't re-sector the drive. If,

however, the Swift drive is in true physical mode, then the lo-level

format will re-sector the drive.

3.  ZBR:  Finally, today's AT interface drives (like the ST-1144A and

ST-3144A) are often Zone Bit Recorded (ZBR). ZBR drives, have variable

sectors per track, depending on the zone of the drive. The outside

tracks, being larger in circumference (i.e. track length is longer),

are able to hold more sectors than the innermost tracks. In this

scenario, it is IMPOSSIBLE to define the drive in CMOS setup with true

physical values. Cylinders and heads, yes.... but not the sectors per

track.  Therefore, these drive are ALWAYS in translation mode and

immune to a re-sectoring lo-level format.  On ZBR AT interface drives

(Seagate, at least... others UNK), the factory defect mapping files

are fully protected, and since the drive is always in translation, the

optimized skewing is also protected.

As to defect management, most AT interface drive's show 0 bytes in bad

sectors under CHKDSK.  This is a courtesy reallocation or "slipping"

of bad sectors by the factory format, and not part of the interface

definition.

There are a few good reasons to consider a lo-level format for a ZBR

AT/IDE drive.  Because a lo-level will "data-scrub" all the sectors,

this may be the only way to delete a corrupted partition record, or

partition record from another operating system, or even a virus

infection.  If a new defect surfaces, maybe from a head slap

(earthquake!), SGATFMT3 is able to find and lock out the offending

sector, provided the defect is not in the ID portion of the sector.

In this method, a kind of mid-level format, the locked out sector will

be found again during the DOS hi-level format and will indicate as

"bytes in bad sectors" at the conclusion.

==========================================================

ABOUT TRANSFER RATES

There seems to be a lot of confusion concerning data transfer rates on

hard disk drives. This is a pity, as this should be a very

straightforward issue. The first thing to do is forget the sales

literature in expressing the practical transfer rate of a drive. The

internal and external transfer ratings are only useful as an estimate

of the maximum bus transfer rate of the area in question. What that

usually means is that those rates are the measure of the speed both

data and commands can be transferred across a given bus in a given

rate of time. For all practical intents and purposes, this is only a

valid for clocking command transfer rates, and data transfer in burst

mode.

For sustained data transfer rate, the bottom line is, the more sectors

that pass under the head in a second, the faster the data comes off of

the drive. To calculate the sustained rate, use this formula :

(512 * Drive RPM * SPT) / (Interleave * 60)

This rating is in Bytes / Second. For Example, a 251 at 3:1 interleave

would transfer data as follow : (512 * 3600 * 17)/(3*60)=174,080

Bytes/second. This is the maximum data transfer rate possible without

caching. To differentiate, and explain failings, you must realize that

the above formula is for IDEAL conditions. Delays can be introduced

by track crossings, head switch time, or, most importantly, how the

system asks for the data.

There is also the system overhead to look at, which can be grouped in

with data inquiry delay. To illustrate the latter, think of the drive

rotating at 3600 RPMs. The host system wants several sectors worth of

information for its spreadsheet. It asks for a sector read. The drive

acknowledges the command. the system waits. The drive steps to the

proper track. The drive reads. The host acknowledges. The host asks

for the next sector. The drive, which has been spinning all this time

as drives do, no longer has its heads over that sector, because the

host didn't ask for data in time. The drive spins. The sector is read,

and so on. This procedure is much faster if the host just asks for a

multiple sector read, as once the data is located, it streams directly

off of the drive. This condition can be masked by the use of buffers,

because the next few data requests can be satisfied by the queue, or

buffer, whether built into the drive controller, or allocated to the

system memory. Both of these schemes anticipate a multiple sector read

beforehand, and fill memory locations with the data from the next few

contiguous sectors. Although this works for the most part, once the

queue is exhausted, we are back to the limitation of the sustained

transfer rate, to be found by the aforementioned formula.

==========================================================

ANATOMY OF A SECTOR

-------------------

The purpose of a track format is to organize a data track into smaller

sequentially numbered blocks called sectors.  The beginning of each

sector is defined by a pre-written identification (ID) field which

contains the Logical sector address plus cylinder and head

information.  The ID field is then followed by a user supplied data

field.

Anatomy of a Sector (17-sector, 512 byte/sector):

Index                                                            Index

ڿ                                                                   ڿ

��                                                                   ��

������������������������������������������������������������������������

� 1 � 2 � 3 � 4 � 5 � 6 � 7 � 8 � 9 �10 �11 �12 �13 �14 �15 �16 �17 �

Gap1                         �   �                                  Gap4

�   �

571 Bytes Total

������������������������������   ��������������������������������������Ŀ

(Field Types:)

�Sync.���ID Field���������Gap2����Data Field�����������������������Gap3Ŀ

�   1   �   2 3 4 5 6 7  �  8 9  � 10 11           12          13  14 15

(Field No.)

Field No.  Bytes  Field              Description

1      13     ID VFO Lock        A field of all zeros to synchronize

the VFO for the ID.

2       1     Sync. Byte         A1h with a dropped clock to notify

the controller that data follows.

3       1     Address Mark       FEh: ID data field follows.

4       2     Cylinder Address   A numerical value in Hex defining

the detent position of the

actuator.

5       1    *Head Number        A numerical value in Hex defining

the head selected.

6       1     Sector Number      A numerical value in Hex defining

the sector for this section of

the rotation.

7       2   **CRC                Cyclic Redundancy Check

information used to verify the

validity of the ID information

field just read.

8       3     Write Turn On      Zeros written during format to

Gap                isolate the write splice created.

This field assures valid reading

of field number seven and allows

the 13 bytes required for data

VFO lock.

9      13     Data Sync.         A field of all zeros to sync the

VFO Lock           VFO for the data field.

10       1     Sync. Byte         A1h with a dropped clock to

notify the controller that data

follows.

11       1     Address Mark       F8h: User data follows.

12     512     Data               User Data.

13       2   **CRC                Cyclic Redundancy Check

information used to verify the

validity of the user data field

just read.

14       3     Write Turn Off     Zeros written during update to

Gap                isolate the write splice created.

This field assures valid reading

of field number 13 and allows the

13 bytes required for VFO lock

for the ID field of the next

sector.

15      15     Inter-Record Gap   A field of 4Eh  which acts

as a buffer between sectors to

allow for speed variation.

Index : This is a signal which occurs once per revolution and it

functions to indicate the physical beginning of the track.

* Head Number : bits 0, 1, 2 = Head Number

bits 3, 4    = '00'

bits 5, 6    = Sector Size = '00'

bit  7       = Bad Block Mark

** CRC : These codes are generated by the controller, and written on

the media during formatting. Data integrity is maintained by

the controller, recalculating and verifying the ID Field check

codes when the ID Field is read. An acceptable polynomial is:

16  12  5

X  +X  +X +1

In the case of the Data Field CRC, instead of two bytes of

Data CRC, the controller may implement a multiple byte Error

Correction Code (ECC) Data Field integrity system.  An ECC

system provides the possibility of data field read correction

as well as read error detection.  The correction/detection

ability is dependent on the code chosen and the controller

implementation.

Gap1 : Provides a head switching recovery period and controller

decision making period, so when switching from one track to

another, sequential sectors may be read without waiting the

entire rotational latency time (additional time may be

required on 1 to 1 controllers by adding a head skew).

Gap2 : This gap follows the CRC bytes of the ID field and continues

to the data field address mark.  Written by the controller, it

provides both a pad to ensure a proper recording and recovery

of the last bits of the ID Field check codes and to allow time

for controller decision making plus a byte for a write splice.

The write splice will be created on the media as soon as the

interface Write Gate is activated when performing a Data Field

update function.

Gap3 : Also known as the inter-record gap, this gap follows the CRC

bytes of the Data area.  In addition to similarities to Gap2,

it also provides a means to accommodate variances in spindle

speeds.  A track may have been formatted while the disk is

running slower than nominal, then write updated with the disk

running faster than normal. Without a gap, or if the gap is

too small, the sync bytes or ID field of the next sector could

be overwritten. The actual size of this padding, initially

provided by the format function, will vary, affected by on the

disk rotational speed variations when the track was formatted

and each time the Data Field is updated.

Gap4 : This is the speed tolerance gap for the entire track.  It is

required to insure that the entire track can be formatted

during an Index Pulse to Index Pulse Track Format operation.

This Preindex gap will vary in actual size, depending on the

disk rotational speed (+-0.5%) and write frequency tolerance

(+-0.01%) at the time of formatting.

==========================================================

About Choosing a Drive Type in an AT:

The drive types for SCSI, RLL, and ESDI interface drives are generally

easy to determine, especially the SCSI drives.

SCSI

Almost all SCSI drives use DRIVE TYPE 0 or NONE, as the host adapter

bios and the drive communicate together to establish the drive

geometry.  The low-level formatting routines are accessed on the host

adapter through DEBUG.  After the low-level format, follow the

instructions for your DOS version for partitioning and system format.

Note:  SCSI drives from the Seagate Wren and Swift families are

already low-level formatted at the factory.

RLL / ESDI

RLL and ESDI drives are usually not represented at all in the internal

drive tables and consequently the controllers for these drives have

onboard a ROM BIOS which either contains its own internal list of

choices for the interface or else provides the ability to dynamically

configure (define) the controller to the specific geometry of the

drive.  In the case of the ESDI interface, the controller gets

parameters directly from the drive with a mode sense equivalent

command.  Unlike the SCSI, the CMOS drive type should start at 0 or

NONE at the start of the installation (low level format through DEBUG

- consult your controller manual for instructions), but it may be

reset to DRIVE TYPE 1 by the controller card.

Many of the older AT's only provided 14 (MFM only) or so drive types

to choose from in the CMOS.  The middle-aged AT's usually have up to

46 (still usually only MFM) types.  Some newer AT's have drive types

which begin to include direct support for the popular RLL and ESDI

drives.  If you have this newer kind of CMOS then by all means pick

the one that matches the drive and DISABLE the controller Bios. (Note:

This may also disable the controller's caching feature). Likewise,

most new machines have a "User Definable" or "Custom" drive type that

can be created and saved in the CMOS, thus providing a standard drive

type.  "User Definable" drive types will usually not work with most

non-MS/PC-DOS applications.

A special note on ESDI and other drives that have more than 1024

cylinders.  Since DOS cannot access cylinders above this 1024 limit, a

translation scheme may be elected in the controller's bios.  As the

number of Logical Block Address (LBAs) is defined as

CYLINDERS*HEADS*SECTORS PER TRACK, translations that equal the same

number of LBAs with the cylinder count below the 1024 limit will be

devised.  The controller bios will need to be ENABLED in order to

utilize translations schemes.  (e.g. Many popular controllers increase

the number of sectors and/or heads and decrease the # of cylinders to

achieve an equivalent number of LBAs.  See your controller manual for

details.)  After low-level formatting, follow the instructions for

your DOS version for partitioning and system format.

AT / IDE

This idea of translation schemes bring us to the AT or IDE (Imbedded

Drive Electronics) interface.  These drives are intelligent in that

they can use the geometry that represents their true physical

parameters or else they can "mimic" other drive geometries (or

translations) that equal or are very close to, but NOT exceeding, the

same number of logical blocks.  (Translated LBA's <= Native LBA's.)

Many AT/IDE drives have physical cylinder counts that are greater than

1024.  Therefore, for DOS users, it is necessary to utilize the

translate feature by using a geometry that keeps the cylinder count

below 1024.

In order of preference, choose the first that fits your system:

1.  Does the CMOS have a drive type that matches your drive?

no?

2.  Does the CMOS have a drive type that has the same number of

formatted megabytes?

no?

3.  Does the CMOS have a "custom" or "user definable" drive type

option you can use?  If so, use a translation geometry to keep the

cylinder count below the DOS 1024 limit.

no?

4.  Do you have the Disk Manager program to provide a software

driven solution?  The Disk Manager will run automatically to perform

the partitioning and system format.

no?

5.  Pick the drive type that comes closest to, but not

exceeding, the formatted capacity of your drive.  The final

formatted capacity of the drive will be equal to the drive type

chosen.

***  Warning! ALL AT drives from Seagate are already low-level

formatted at the factory.

MFM (ST412 interface)

Finally, the MFM drives and their associated drive types are next. If

the internal drive type table lists the exact geometry, great. If not,

then check to see if a "Custom" or "User Definable" CMOS option is

available.  Also, some AT 16-bit MFM controllers provide an onboard

BIOS which will allow the unique geometry of the drive to be

dynamically configured (our Seagate ST21M/22M MFM controllers have

this VALUABLE feature).  Otherwise, a drive type match that is close

but not exceeding either the cylinder or head values is the only

choice left.  An exact match in the head count is definitely preferred

when getting a "close" match.

When there is no direct match in the internal drive type tables, a

partitioning program may be needed to provide a software driven

translation solution in order to achieve full capacity.  Keep in mind

that the drive will only format out to the capacity of the chosen

drive type when not using partitioning software.  In the event that

the ST412 Interface drive has more than 1024 cylinders, a partitioning

program will be needed in order to achieve full capacity.

==========================================================

GLOSSARY OF DISC DRIVE TERMINOLOGY

ADDRESS

(physical) A specific location in memory where a unit record, or

sector, of data is stored. To return to the same area on the disc,

each area is given a unique address consisting of three components:

cylinder, sector, and head. CYLINDER ADDRESSING is accomplished by

assigning numbers to the disc's surface concentric circles

(cylinders). The cylinder number specifies the radial address

component of the data area. SECTOR ADDRESSING is accomplished by

numbering the data records (sectors) from an index that defines the

reference angular position of the discs. Index records are then

counted by reading their ADDRESS MARKS. Finally, HEAD ADDRESSING is

accomplished by vertically numbering the disc surfaces, usually

starting with the bottom-most disc data surface. For example, the

controller might send the binary equivalent of the decimal number

610150 to instruct the drive to access data at cylinder 610, sector

15, and head 0.

BIT DENSITY

Expressed as "BPI" (for bits per inch), bit density defines how

many bits can be written onto one inch of a track on a disc surface.

It is usually specified for "worst case", which is the inner track.

Data is the densest in the inner tracks where track circumferences are

the smallest.

BIT JITTER

The time difference between the leading edge of read and the

center of the data window.

BIT SHIFT

A data recording effect, which results when adjacent 1's written

on magnetic discs repel each other. The "worst case" is at the inner

cylinder where bits are closest together. BIT SHIFT is also called

pulse crowding.

BLOCK

A group of BYTES handled, stored and accessed as a logical data

unit, such as an individual file record. Typically, one block of data

is stored as one physical sector of data on a disc drive.

CLOSED LOOP

A control system consisting of one or more feedback control loops

in which functions of the controlled signals are combined with

functions of the command to maintain prescribed relationships between

the commands and the controlled signals.

This control technique allows the head actuator system to detect

and correct off-track errors. The actual head position is monitored

and compared to the ideal track position, by reference information

either recorded on a dedicated servo surface, or embedded in the

inter-sector gaps. A position error is used to produce a correction

signal (FEEDBACK) to the actuator to correct the error. See TRACK

FOLLOWING SERVO.

CLUSTER SIZE

Purely an operating system function or term describing the number

of sectors that the operating system allocates each time disc space is

needed.

CODE

A set of unambiguous rules specifying the way which digital data

is represented physically, as magnetized bits, on a disc drive. One of

the objectives of coding is to add timing data for use in data

reading. See DATA SEPARATOR, MFM and RLL.

COERCIVITY

A measurement in units of orsteads of the amount of magnetic

energy to switch or "coerce" the flux change (di-pole) in the magnetic

recording media.

CONTROLLER

A controller is a printed circuit board required to interpret data

access commands from host computer (via a BUS), and send track

seeking, read/write, and other control signals to a disc drive. The

computer is free to perform other tasks until the controller signals

DATA READY for transfer via the CPU BUS.

CYCLIC-REDUNDANCY-CHECK

(CRC). Used to verify data block integrity. In a typical scheme, 2

CRC bytes are added to each user data block. The 2 bytes are computed

from the user data, by digital logical chips. The mathematical model

is polynomials with binary coefficients. When reading back data, the

CRC bytes are read and compared to new CRC bytes computed from the

read back block to detect a read error. The read back error check

process is mathematically equivalent to dividing the read block,

including its CRC, by a binomial polynomial. If the division remainder

is zero, the data is error free.

CYLINDER

The cylindrical surface formed by identical track numbers on

vertically stacked discs. At any location of the head positioning arm,

all tracks under all heads are the cylinder. Cylinder number is one of

the three address components required to find a specific ADDRESS, the

other two being head number and sector number.

DAISY CHAIN

A way of connecting multiple drives to one controller. The

controller drive select signal is routed serially through the drives,

and is intercepted by the drive whose number matches. The disc drives

have switches or jumpers on them which allow the user to select the

drive number desired.

DATA

Information processed by a computer, stored in memory, or fed into

a computer.

DATA ACCESS

When the controller has specified all three components of the

sector address to the drive, the ID field of the sector brought under

the head by the drive is read and compared with the address of the

target sector. A match enables access to the data field of the sector.

DATA ADDRESS

To return to the same area on the disc, each area is given a

unique address consisting of the three components: cylinder, head and

sector. HORIZONTAL: accomplished by assigning numbers to the

concentric circles (cylinders) mapped out by the heads as the

positioning arm is stepped radially across the surface, starting with

0 for the outermost circle. By specifying the cylinder number the

controller specifies a horizontal or radial address component of the

data area. ROTATIONAL: once a head and cylinder have been addressed,

the desired sector around the selected track of the selected surface

is found by counting address marks from the index pulse of the track.

Remember that each track starts with an index pulse and each sector

starts with an address mark. VERTICAL: assume a disc pack with six

surfaces, each with its own read/write head, vertical addressing is

accomplished by assigning the numbers 00 through XX to the heads, in

consecutive order. By specifying the head number, the controller

specifies the vertical address component of the data area.

DATA FIELD

The portion of a sector used to store the user's DIGITAL data.

Other fields in each sector include ID, SYNC and CRC which are used to

locate the correct data field.

DATA SEPARATOR

Controller circuitry takes the CODED playback pulses and uses the

timing information added by the CODE during the write process to

reconstruct the original user data record. See NRZ, MFM, and RLL.

DATA TRACK

Any of the circular tracks magnetized by the recording head during

data storage.

DATA TRANSFER RATE

(DTR). Speed at which bits are sent: In a disc storage system, the

communication is between CPU and controller, plus controller and the

disc drive. Typical units are bits per second (BPS), or bytes per

second, e.g., ST506/412 INTERFACE allows 5 Mbits/sec. transfer rate.

DEDICATED SERVO SYSTEM

A complete disc surface is dedicated for servo data.

DISC/PLATTER

For rigid discs, a flat, circular aluminum disc substrate, coated

on both sides with a magnetic substance (iron oxide or thin film metal

media) for non-VOLATILE data storage. The substrate may consist of

metal, plastic, or even glass. Surfaces of discs are usually

lubricated to minimize wear during drive start-up or power down.

DROP-IN/DROP-OUT

Types of disc media defects usually caused by a pin-hole in the

disc coating. If the coating is interrupted, the magnetic flux between

medium and head is zero. A large interruption will induce two

extraneous pulses, one at the beginning and one at the end of the

pin-hole (2 DROP-INs). A small coating interruption will result in no

playback from a recorded bit (a DROP-OUT).

ECC

ERROR CORRECTION CODE: The ECC hardware in the controller used to

interface the drive to the system can typically correct a single burst

error of 11 bits or less. This maximum error burst correction length

is function of the controller. With some controllers the user is

allowed to the select this length. The most common selection is 11.

ELECTRO-STATIC DISCHARGE

(ESD) An integrated circuit (CHIP) failure mechanism. Since the

circuitry of CHIPs are microscopic in size, they can be damaged or

destroyed by small static discharges. People handling electronic

equipment should always ground themselves before touching the

equipment. Electronic equipment should always be handled by the

chassis or frame. Components, printed circuit board edge connectors

should never be touched.

EMBEDDED SERVO SYSTEM

Servo data is embedded or superimposed along with data on every

cylinder.

FCI

(FLUX CHANGES PER INCH): Synonymous with FRPI (flux reversals per

inch). In MFM recording 1 FCI equals 1 BPI (bit per inch). In RLL

encoding schemes, 1 FCI generally equals 1.5 BPI.

FILE ALLOCATION TABLE

FAT: What the operating systems uses to keep track of which

clusters are allocated to which files and which are available for use.

FAT is usually stored on Track-0.

FIRMWARE

A computer program written into a storage medium which cannot be

accidentally erased, e.g., ROM. It can also refer to devices

containing such programs.

FIXED DISC

A disc drive with discs that cannot be removed from the drive by

the user, e.g., WINCHESTER DISC DRIVE.

FLUX CHANGE

Location on the data track, where the direction of magnetization

reverses in order to define a 1 or 0 bit.

FLUX CHANGES PER INCH

(FCI). Linear recording density defined as the number of flux

changes per inch of data track.

FM

Frequency modulation CODE scheme, superceded by MFM, which is

being superceded by RLL.

FORMAT

The purpose of a format is to record "header" data that organize

the tracks into sequential sectors on the disc surfaces. This

information is never altered during normal read/write operations.

Header information identifies the sector number and also contains the

head and cylinder ADDRESS in order to detect an ADDRESS ACCESS error.

FORMATTED CAPACITY

Actual capacity available to store user data. The formatted

capacity is the gross capacity, less the capacity taken up by the

overhead data used in formatting the discs. While the unformatted size

may be 24 M bytes, only 20 M bytes of storage may actually be

available to the user after formatting.

FPI

(flux changes per inch), also FRPI, the number of Flux Reversals

per inch.

GAP

1. FORMAT: Part of the disc format. Allows mechanical

compensations (e.g. spindle motor rotational speed variations) without

the last sector on a track overwriting the first sector. 2. HEAD: An

interruption in the permeable head material, usually a glass bonding

material with high permeability, allowing the flux fields to exit the

head structure to write / read data bits in the form of flux changes

on the recording media.

GAP LENGTH

Narrowing the head gap length achieves higher bit density because

the lines of force magnetize a smaller area where writing data in the

form of flux changes on the recording media.

GAP WIDTH

The narrower the gap width, the closer the tracks can be placed.

Closer track placement results in higher TPI.

GCR

GROUP CODE ENCODING. Data encoding method.

GUARD BAND

1. Non-recorded band between adjacent data tracks, 2. For closed

loop servo drives, extra servo tracks outside the data band preventing

the Carriage Assembly from running into the crash stop.

HARD ERROR

An error that occurs repeatedly at the same location on a disc

surface. Hard errors are caused by imperfections in the disc surface,

called media defects. When formatting hard disc drives, hard error

locations, if known, should be spared out so that data ia not written

to these locations. Most drives come with a hard error map listing the

locations of any hard errors by head, cylinder and BFI (bytes from

index - or how many bytes from the beginning of the cylinder).

HARD ERROR MAP

Also called defect map, bad spot map, media map. Media defects are

avoided by deleting the defective sectors from system use, or

assigning an alternative track (accomplished during format operation).

The defects are found during formatting, and their locations are

stored on a special DOS file on the disc, usually on cylinder 0.

HEAD

An electromagnetic device that can write (record), read

(playback), or erase data on magnetic media. There are three types:

Head Type BPI TPI Areal density Monolithic 8000 450 3.6 X 10 to 6th

Composition 12000 1000 12 X 10 to 6th Thin-film 25000 1500 37.5 X 10

to 6th

HEAD SLAP

Similar to a head crash but occurs while the drive is turned off.

It usually occurs during mishandling or shipping. Head slap can cause

permanent damage to a hard disc drive. See HEAD CRASH.

ID FIELD

The address portion of a sector. The ID field is written during

the Format operation. It includes the cylinder, head, and sector

number of the current sector. This address information is compared by

the disc controller with the desired head, cylinder, and sector number

before a read or write operation is allowed.

INDEX

(PULSE): The Index Pulse is the starting point for each disc

track. The index pulse provides initial synchronization for sector

addressing on each individual track.

INDEX TIME

The time interval between similar edges of the index pulse, which

measures the time for the disc to make one revolution. This

information is used by a disc drive to verify correct rotational speed

of the media.

INTERFACE

The protocol data transmitters, data receivers, logic and wiring

that link one piece of computer equipment to another, such as a disc

drive to a controller or a controller to a system bus. Protocol means

a set of rules for operating the physical interface, e.g., don't read

or write before SEEK COMPLETE is true.

INTERLEAVE FACTOR

The ratio of physical disc sectors skipped for every sector

actually written.

INTERLEAVING

The interleave value tells the controller where the next logical

sector is located in relation to the current sector. For example, an

interleave value of one (1) specifies that the next logical sector is

physically the next sector on the track. Interleave of two (2)

specifies every other physical sector, three (3) every third sector

and so on. Interleaving is used to improve the system throughout based

on overhead time of the host software, the disc drive and the

controller; e.g., if an APPLICATION PROGRAM is processing sequential

logical records of a DISC FILE in a CPU time of more than one second

but less than two, then an interleave factor of 3 will prevent wasting

an entire disc revolution between ACCESSES.

LATENCY

(ROTATIONAL) The time for the disc to rotate the accessed sector

under the head for read or write. On the average, latency is the time

for half of a disc revolution.

LOW LEVEL FORMAT

The first step in preparing a drive to store information after

physical installation is complete. The process sets up the "handshake"

between the drive and the controller. In an XT system, the low level

format is usually done using DOS's debug utility. In an AT system, AT

advanced diagnostics is typically used. Other third party software may

also be used to do low level format on both XTs and ATs.

MEDIA DEFECT

A media defect can cause a considerable reduction of the read

signal (missing pulse or DROP-OUT), or create an extra pulse

(DROP-IN). See HARD ERROR MAP.

MEGABYTE

One million bytes (exactly 1,000,000 bytes). Abbreviation: MB or

Mbyte.

MODIFIED FREQUENCY MODULATION

(MFM). A method of recording digital data, using a particular CODE

to get the flux reversal times from the data pattern. MFM recording is

self-clocking because the CODE guarantees timing information for the

playback process. The controller is thus able to synchronize directly

from the data. This method has a maximum of one bit of data with each

flux reversal. (See NRZ, RLL).

NRZ

NON-RETURN TO ZERO 1) User digital data bits; 2) A method of

magnetic recording of digital data in which a flux reversal denotes a

one bit, and no flux reversal a zero bit, NRZ recording requires an

accompanying synchronization clock to define each cell time unlike MFM

or RLL recording). No Seagate drives use NRZ recording methods.

PRECOMPENSATION

Applied to write data by the controller in order to partially

alleviate bit shift which causes adjacent 1's written on magnetic

media physically to move apart. When adjacent 1's are sensed by the

controller, precompensation is used to write them closer together on

the disc, thus fighting the repelling effect caused by the recording.

Precompensation is only required on some oxide media drives.

READ

To access a storage location and obtain previously recorded data.

RECALIBRATE

Return to Track Zero. A common disc drive function in which the

heads are returned to track 0 (outermost track).

REDUCED WRITE CURRENT

A signal input (to some older drives) which decreases the

amplitude of the write current at the actual drive head. Normally this

signal is specified to be used during inner track write operations to

lessen the effect of adjacent bit "crowding." Most drives today

provide this internally and do not require controller intervention.

RESOLUTION

With regards to magnetic recording, the band width (or frequency

response) of the recording heads.

RLL

(RUN LENGTH LIMITED CODE). 1) A method of recording digital data,

whereby the combinations of flux reversals are coded/decoded to allow

greater than one (1) bit of information per flux reversal. This

compaction of information increases data capacity by approximately 50

percent; 2) a scheme of encoding designed to operate with the ST412

interface at a dial transfer rate of 7.5 megabit/sec. The technical

name of the specific RLL CODE used is "two, seven".

ROM

(READ ONLY MEMORY) A chip that can be programmed once with bits of

information. This chip retains this information even if the power is

turned off. When this information is programmed into the ROM, it is

called burning the ROM.

ROTATIONAL SPEED

The speed at which the media spins. On a 5-1/4 or 3-1/2"

Winchester drive it is usually 3600 rpm.

SECTOR

A sector is a section of a track whose size is determined by

formatting. When used as an address component, sector and location

refer to the sequence number of the sector around the track.

Typically, one sector stores one user record of data. Drives typically

are formatted from 17 to 26 sectors per track. Determining how many

sectors per track to use depends on the system type, the controller

capabilities and the drive encoding method and interface.

SECTOR-SLIP

Sector-slip allows any sector with a defect to be mapped and

bypassed. The next contiguous sector is given that sector address.

SERVO TRACK

A prerecorded reference track on the dedicated servo surface of a

closed-loop disc drive. All data track positions are compared to their

corresponding servo track to determine "off-track/on-track" position.

SKEWING

Some low-level formatting routines may ask for a Head and/or

Cylinder Skew value. The value will represent the number of sectors

being skewed to compensate for head switching time of the drive and/or

track-to-track seek time allowing continuous read/write operation

without losing disk revolutions.

SOFT ERROR

A bit error during playback which can be corrected by repeated

attempts to read.

TRACK

The radial position of the heads over the disc surface. A track is

the circular ring traced over the disc surface by a head as the disc

rotates under the heads.

TRACK FOLLOWING SERVO

A closed-loop positioner control system that continuously corrects

the position of the disc drive's heads by utilizing a reference track

and a feedback loop in the head positioning system. See also CLOSED

LOOP.

TRACK ZERO

Track zero is the outermost data track on a disc drive. In the ST

506 INTERFACE, the interface signal denotes that the heads are

positioned at the outermost cylinder.

VOICE COIL MOTOR

An electro-magnetic positioning motor in the rigid disk drive

similar to that used in audio speakers. A wire coil is placed in a

stationary magnetic field. When current is passed through the coil,

the resultant flux causes the coil to move. In a disc drive, the

CARRIAGE ASSEMBLY is attached to the voice coil motor. Either a

straight line (linear) or circular (rotary) design may be employed to

position the heads on the disc's surface.

WEDGE SERVO SYSTEM

A certain part of each CYLINDER contains servo positioning data.

Gap spacing between each sector contains servo data to maintain

position on that cylinder.

WRITE CURRENT

The optimum HEAD write current necessary to saturate the magnetic

media in a cell location.

ZBR (Zone Bit Recording)

Trademark of Seagate Technology. A media optimization technique

where the number of sectors per track is dependent upon the cylinder

circumference. E.G. tracks on the outside cylinders have more sectors

per track than the inside cylinders. The ZBR format is only done at

the factory. These drives should not be low-level formatted by the

end-user.

==========================================================

Other

-----

Available on the Seagate Tech Support BBS (408)438-8771:

Specifications and jumper drawings for all Seagate Disc Drives and

Controllers.

Reprints of Installation Guides.

Troubleshooting essays.

FINDTYPE - Utility which displays bios drive type table and matches a

Seagate model to the best drive type.  Also prints complete

specifications lists and much more!

FINDINIT - Utility for Seagate controllers and host adapters that have

onboard bios, namely ST01, ST02, ST05X, ST11M, ST11R, ST21M, ST21R,

ST22M, and ST22R.  Queries the system to determine bios memory address

and initiates controller bios lo-level format.

FLASHLED - TSR utility which shows disc drive activity on one of the

keyboard LED's.

DESK REFERENCE - Hypertext data system for all Seagate products,

troubleshooting, other OEM phone numbers and much, much more. A must

for dealers who do a fair amount of support for Seagate products.

==========================================================

LICENSE AGREEMENT

Seagate provides the accompanying object code software ("Software")

and nonexclusively licenses its use on the following terms and

conditions. The Software is copyrighted by Seagate.  YOU ASSUME FULL

RESPONSIBILITY FOR THE SELECTION OF THE SOFTWARE TO ACHIEVE YOUR

INTENDED PURPOSES, FOR THE PROPER INSTALLATION AND USE.  SEAGATE DOES

NOT WARRANT THAT THE SOFTWARE WILL MEET YOUR REQUIREMENTS, THAT THE

SOFTWARE IS FIT FOR ANY PARTICULAR PURPOSE OR THAT THE USE OF THE

SOFTWARE WILL BE ERROR FREE.  SEAGATE EXPRESSLY DISCLAIMS ALL

WARRANTIES, WHETHER ORAL OR WRITTEN, EXPRESSED OR IMPLIED, INCLUDING

WITHOUT LIMITATION WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A

PARTICULAR PURPOSE.  IN NO EVENT WILL SEAGATE BE LIABLE TO YOU, YOUR

CUSTOMERS OR OTHER USERS FOR ANY INDIRECT, INCIDENTAL, CONSEQUENTIAL,

SPECIAL OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE

USE OR INABILITY TO USE THE SOFTWARE.

End of License agreement.

-=EOF: SGATFMT3.DOC=-