Floppy Tool Kit, Version 1.0

William Luitje

20 January 1994

This  is  a collection of 5 small utilities  which  will

help  you  use  your floppy disks  better.    Since  you

probably  worry more about how to get better  speed  and

storage  out  of your hard disk,  this is not  likely  a

matter of pressing concern to you.   However,   floppies

are going to be used for quite a while and by spending a

few minutes with these utilities you can make things  go

a little bit faster each time you use a floppy.

FLOPPARM

~~~~~~~~

Floppies  have  been  around a long time and  have  been

improved since they first came out, not only in capacity

but  also  in their mechanical  speed.    Unfortunately,

system software can't tell what the best speed to  drive

a  floppy is and must be compatible with old drives  and

so uses conservatively slow parameters.   When accessing

a  floppy,   the BIOS uses a table called the Disk  Base

Table for these parameters.   This table can be found by

looking  at interrupt vector 0X1E.  Since this table  is

usually  set  up in RAM by DOS,  it is an easy thing  to

modify  it for modern,  fast drives.   FLOPPARM will  do

this for you conveniently; simply run it with one of the

three arguments SLOW,  MEDIUM, or FAST.   The parameters

which are affected are:

� step rate and head unload time, which are parameters

of the disk controller chip.

� head settle time,  which is the amount of time taken

after stepping from one track to another to wait for

the head mechanism to stop vibrating.

� motor  startup time,  which  is the amount  of  time

taken to allow the spindle motor speed to  stabilize

after it first starts up.

SLOW  is  provided  for compatibility  with  really  old

drives;  MEDIUM is somewhat faster than DOS 5;  and FAST

is  pedal  to  metal!  When run,  FLOPPARM  displays the

parameters used.  If run with no or an illegal argument,

FLOPPARM  will  display  a usage hint  and  the  current

values of the parameters.

FLOPPARM also allows you to specify your own  parameters

by using the CUSTOM argument followed by an equals  sign

and  the four parameters (step rate,  head unload  time,

head  settle time and motor startup time)  separated  by

commas  (no  spaces allowed).   For  example  a  command

equivalent to FLOPPARM MEDIUM would be

flopparm custom=13,15,1,2

Legal values for step rate and head unload time are from

0   to  15.    Legal values for  head  settle  time  (in

milliseconds)   and  motor startup time (in  1/8s  of  a

second) are from 0 to 255

Note that FLOPPARM is not a TSR and does not take up any

DOS memory;  it simply modifies a table which is already

created by DOS. Once you decide which parameters to use,

just  put  a line invoking FLOPPARM in your AUTOEXEC.BAT

file and enjoy a modest increase in floppy speed.

FLOPTEST

~~~~~~~~

How  can  you  tell if you have set  your  floppy  drive

parameters too fast?  I've included a utility which will

seek  to  every  track of the disk,  read a  sector  and

report any errors it finds.   The seek pattern is called

a "butterfly test"  since it reads the first track, then

the last track, then the second track,  then the next to

last  track,  etc.   This will really give the  drive  a

workout!

This  test requires a formatted floppy which has no  bad

sectors  (FLOPTEST can't tell if an error is due to  the

drive not working properly or due to a bad disk).  It is

a read only test so it shouldn't change any data  on the

disk.    There  are two parameters:   one is  the  drive

letter  and the second is the number of passes  to  run.

If  you  get any errors and your floppy is ok  then  you

should  back  off  one  or  more  of  the  floppy  drive

parameters and test again. Here is an example

floptest a 20

If  you  are fortunate enough to have more than  2  disk

drives  (this applies both to FLOPTEST and to the  other

utilities in this package),  you can refer to the  third

drive as c and the fourth drive as d, even if you have a

hard drive that goes by either of those names.   I  know

this  is  confusing  but this test operates at the  BIOS

level  and has no idea what  letter DOS has assigned  to

your  third  and  fourth floppies.   I  had a choice  of

confusing  people who had more than 2 floppy  drives  or

confusing everybody by making you specify the drive with

a number rather than a letter.  That's life.

You  can  also use FLOPTEST to see if a floppy disk  has

any  bad  sectors.   If you run it with only  the  drive

letter  specified,  then it will attempt to  read  every

sector  and  report any failures.  Since FLOPTEST  makes

only  one attempt to read each sector,  it will warn you

if a sector is getting weak.

FLOPSKEW

~~~~~~~~

The  most  important improvement you can make  in  using

your  floppy  drives  is  to  format  your  disks  using

FDFORMAT  by  Christoph  Hochst�tter  or ATFMT  by  Oleg

Kibirev.    These programs allow almost complete control

over  how your disks are formatted.  Among other things,

you can put up to 820K on 5�"  DSDD disks,  1.48  meg on

5�"  HD disks and 1.7 meg on 3�" HD disks.   In terms of

speeding up floppy disk transfers,  these programs allow

you to control the way sectors are arranged on  a track.

This concept is  called  sector  sliding by FDFORMAT and

sector spinning by ATFMT; in the US it's known as sector

skewing.   Basically,  the idea is  that since  there is

some mechanical delay in changing heads or tracks it may

be  desirable to put the first sector of a new  head  or

track "later" in the track so the computer doesn't  have

to  wait for the floppy to make  a  complete  revolution

before  that sector can be read.   See  these  programs'

documentation for more details on how  this works.   The

time  savings  when reading or  writing  a floppy can be

substantial  when you format it using the  optimal  skew

parameters  and,   as an added benefit,   formatting  is

faster, too.

You  can  specify  head  skew and  track  skew  in  both

FDFORMAT   and   ATFMT  by  the  X  and  Y   parameters,

respectively.    The DOS FORMAT program does not support

sector  skewing  at  all;  this is equivalent  to  using

FDFORMAT  or ATFMT with X and Y both set to zero.    But

what values should you use for these parameters?   Well,

when FDFORMAT first introduced this feature,  I  wrote a

batch  file  which  formatted a test disk  with  various

combinations of the head and sector skew parameters  and

then  measured how long it took to copy a large file  to

the floppy.   It took about an hour to run and tested 16

combinations of X and Y values for just one disk format.

FLOPSKEW  does  the same job in just a few  seconds  but

tests even more combinations of parameter values.

To use FLOPSKEW,  first decide which setting of FLOPPARM

to  use and then run FLOPPARM with that  parameter since

the  step  rate  and head settle time will  both  affect

skewing.   Then, format  a disk of the type and capacity

for   which  you  want  to  find  the  optimal   skewing

parameters  with  the  X and  Y  parameters both  0  and

interleave  1.    Although FLOPSKEW is a non-destructive

test,  you should use a freshly formatted disk to ensure

that these  parameters are set correctly,  otherwise the

test will give strange results.   If you don't specify X

and Y,  FDFORMAT and ATFMT default to the correct values

except  that they both must use an interleave of  2  for

the 5�" 1.44  meg and 3�" 1.7  meg formats (and for this

reason  it doesn't make sense to use sector skewing with

these formats). DOS FORMAT always formats with no sector

skewing and interleave 1.

Then,  simply run FLOPSKEW with the drive letter of  the

floppy you want to test as the only parameter.   Here is

an example  command and its results for a double density

(720K) 3�" disk on my system:

flopskew b

Find Optimal Head & Track Skew, Ver 1.0

9 Sectors/Track, 80 Tracks, 2 Sides

Head Skew Times (milliseconds):

X:       0   1   2   3   4   5   6   7   8

Time:   32  54  74  95 116 137 158 178 199

Track Skew Times (milliseconds):

Y:       0   1   2   3   4   5   6   7   8

Time:  232  53  74  95 116 137 158 178 199

Interleave:  1

Minimum skewing parameters:  X=0, Y=1

Recommended skewing parameters:  X=1, Y=2

First  you  are shown the format of the  test  diskette,

then  some  test  results and  finally  the  recommended

skewing  parameters.    If  you are interested  in  some

technical  details about the test you  should  read  the

next few paragraphs; otherwise you can skip ahead to the

next  section.   Just use the recommended parameters  to

format your disks and expect your diskette throughput to

become about 30% faster.

Two  tests are run to find the best values for the  Head

Skew  parameter (X)  and the Track Skew  parameter  (Y).

The  head skew test is run by reading the last sector of

the  first  track using head 0 and  then  measuring  how

long, in milliseconds, it takes to read the first sector

of  that  same  track  with  head 1.   (If the  disk  is

formatted with only one side this test is not run.)  The

track skew test is run by reading the last sector of the

first  track  using head 1  (if testing a  double  sided

disk)  and then  measuring how long it takes to read the

first  sector of the second track using head 0.   In the

results above, the second line tells how long this took,

in milliseconds,  for each of  the sectors on the second

track read.

Let's look at the example data to see what we can learn.

First,  notice that the times to read different  sectors

in the head skew test increase by about 22 milliseconds.

This  is just the time it takes to read one  sector  for

this format.  The formula is

sector read time(ms) = 1000 * 60sec/min * rpm / sectors

For  this format the drive rotates at 300  rpm and there

are 9 sectors so the time to read a sector works out  to

22.2   milliseconds.    This formula can be modified  to

figure the time to read an entire track by dropping  the

final division by sectors.  In this case, it is 200 ms.

To  determine the  best values for X and Y,  the program

finds  the smallest time and the corresponding number of

sectors skipped from the line above.  The data show that

for my system no sectors should be skipped when changing

heads  so  the optimal head skew parameter,  X,   is  0.

Typical values are 0 or 1 for most systems.  Now look at

the  results  for the track skewing test;  the  time  to

change tracks without skipping any sectors is 232!  This

is  the  time  it takes to read a whole track  plus  the

sector  plus some overhead.   In this case the 0  sector

skew  provided by DOS format and the default values used

by  FDFORMAT  and ATFMT yield a big waste of  time  when

reading  or writing multiple tracks.   On my system  the

optimal track skew parameter,  Y, is 1.   Typically,  it

ranges from 1 to 3 on the systems I have looked at.

Now  notice  that the program recommends values for  the

skew  parameters  which are 1 greater than  the  optimal

values.  This is because the penalty for choosing a skew

value which is too small is severe:   you have to wait a

whole revolution,   in this case 200  ms, when  changing

tracks.     On  the other hand, the penalty for having a

skew value which is one too big is only 22  ms.    Since

some  software may introduce  an additional  delay  when

changing  tracks,   it is a safe bet to be  conservative

when  choosing the  skew values.  If you  regularly  use

floppies  on two computers you should  run  FLOPSKEW  on

both  of them and choose the higher of  the  recommended

skew parameters.

Take the values of X and Y found by the above  procedure

and use them to format your floppies in the future.   To

make this easy, you can create a batch file, 4DOS or CED

alias,   or  FDFORMAT or ATFMT configuration  file  with

those parameters in it.  Any time you format a floppy or

read  or  write  a floppy formatted this way it will  go

faster.

How much faster?   Well,  assuming the most likely  case

that  only  track seek is a problem and that the  sector

skewing  is  only  off by 1 or 2,   then  on average the

computer  has  to wait for a full disk revolution  every

second  track it reads.   Eliminate the wait and you can

reduce  the time spent reading and writing by 1/3.  When

formatting  a diskette, skewing only affects  the verify

operation so the time reduction is somewhat less,  about

20%.

FLOPRPM

~~~~~~~

If you are having trouble reading or writing  diskettes,

especially with a large number of sectors per track, the

problem may lie with the speed at which the drive spins.

FLOPRPM will test this condition for you.   Simply place

a  formatted disk in the drive you want to test and  run

FLOPRPM with the drive letter as the only argument.  The

program  will  then test and display the  rotation  rate

continuously  until you press a key.   Normally,  all PC

floppy  drives  will  run within an RPM or  two  of  300

except for 5�"  HD drives which run at 360.   If you are

having  problems  and the speed of your drive is off  by

more than that,  you may be able to get better operation

by adjusting the speed of your drive.  Unfortunately, no

general guidelines  can be given about how  to  do  that

because drives differ in their design.

FLOPINFO

~~~~~~~~

This  program simply reads the existing format of a disk

and  displays  it.   With all of the  different  formats

supported  by FDFORMAT and ATFMT it's nice to know  what

format a particular disk is.   Also, since some of these

formats require a TSR to read, you might want to know if

a  particular floppy can be read on another system which

doesn't have a copy of that TSR available.  Or,  perhaps

you  want  to quickly find out whether a floppy is HD or

DD.

FLOPINFO takes one parameter, the drive letter.  Here is

an example command  and response,  again  for a 720K 3�"

disk

flopinfo b

Floppy Disk Information, Ver. 1.0 by luitje@m-net.arbornet.org

OEM Name: IBM  3.3, 512 Bytes/Sector, 9 Sectors/Track, 80 Tracks, 2 Sides

2 FATs, 1 Reserved Sectors, 112 Directory Entries

Total Capacity is 730112

Registration

~~~~~~~~~~~~

While this software may be freely copied and used,  I do

request  that you register it if you use it.   Just send

me  either  a postcard or some e-mail telling me who you

are and where you are and what you like or dislike about

the  program.  This is only partially an ego massage for

the author.   I  enjoy writing useful utilities and have

several other pieces of freeware out floating around the

net  so  I  would  like feedback about which of them  is

worth expanding and supporting.

This software is intended to be safe to use.   All tests

and information programs are read only.   About the only

problem  you  might have is that if you set  the  floppy

drive  parameters  too fast you may get  read  or  write

errors.    Never  the less,  in using this software  you

assume all risk should anything go wrong.  I  think that

is pretty unlikely, but, hey, sometimes it rains fish!

William Luitje

2677 Wayside Drive

Ann Arbor, MI 48103

USA

Internet:  luitje@m-net.arbornet.org or

luitje@m-net.ann-arbor.mi.us