����������������������������������������������������������������������������ͻ

�        HWiNFO - Copyright (c)1995-98 RealSoft, All Rights Reserved.        �

� �������������������������������������������������������������������������� �

�                # Version 4.3.4 ; Last Update: May-11-1998 #                �

����������������������������������������������������������������������������ͼ

������������������������������������������Ŀ

�  USE THIS PROGRAM ONLY AT YOUR OWN RISK  �

��������������������������������������������

CONTENTS

1.  Intro

2.  Updates and download

3.  Requirements

4.  Utilities distributed with HWiNFO

5.  Registration

6.  Creating a logfile

7.  Limitations

8.  Running HWiNFO under V86 Memory Managers

9.  Resetting the CPU

10.  HWiNFO Crash

11.  Secondary Video Info

12.  PCI chipsets comparison

13.  IDE Disk Transfer Modes

14.  Miscellaneous notes

15.  Some abbreviations

16.  Codenames

17.  Thanks

18.  Disclaimer

## 1. Intro ##################################################################

HWiNFO is a *POWERFUL* hardware detection program. It is simple, quick and

on most systems very efficient. It "tries" to give only important information

about all the hardware devices in your computer so that you need not look

inside the PC to see what's there.

## 2. Updates and download ###################################################

The newest version is always (immediately) available at:

My homepage   - http://www.svf.stuba.sk/~malik/hwinfo.html

SAC ftp       - ftp.elf.stuba.sk/pub/pc/utilmisc

http://ftp.elf.stuba.sk/packages/pub/pc/utilmisc

SAC BBS       - +421 7 5438 232   (19,200 Baud)

+421 7 5438 295   (28,800 Baud)

At other sitez/BBS later...

SimTel        - ftp.simtel.net/pub/simtelnet/msdos/sysinfo

SAC mirrors   - ftp.uakom.sk/pub/mirrors/sac/utilmisc

ftp.intec.sk/sac/utilmisc

ftp.vse.cz/pub/msdos/SAC/pc/utilmisc

ftp.cs.tu-berlin.de/pub/msdos/mirrors/stuba/pc/utilmisc

ftp-beer.bke.hu/vol1/pub/sac/pc/utilmisc

ftp.pwr.wroc.pl/pub/pc/sac/utilmisc

cert.unisa.it/pub/PC/SAC/utilmisc

ftp2.itb.it/pub/PC/SAC/utilmisc

ftp.nsysu.edu.tw/PC/SAC/utilmisc

192.217.228.45/SAC/pub/pc/utilmisc

ftp.comp.hkbu.edu.hk/pub/sac/utilmisc

ftp.cdrom.com/pub/sac/utilmisc

## 3. Requirements ###########################################################

- cca. 410 (+50) KBytes of free system memory

Well, that's all. I hope it will work on all machines.

## 4. Utilities distributed with HWiNFO ######################################

There are four utilities distributed along with HWiNFO:

GETBIOS.COM     This utility retrieves all BIOS images and saves them to

files <BIOSseg>.ROM

This program doesn't run in V86 mode in order to avoid

BIOS relocation.

GETDDC.COM      This utility saves VESA Display Data Channel data to file

DDC.DAT if your video card and monitor support DDC

GETPCI.EXE      This utility saves all PCI devices data into files

<VendorID><DeviceID>.PCI

GETPNPRS.EXE    This utility saves resource data for all Plug-and-Play

capable devices into files PNP<CSN>.RES

These utilities are free and are designed to help me improve HWiNFO if

you send me the results.

## 5. Registration ###########################################################

HWiNFO is shareware, meaning that you can test the program for free whilst

checking if it is useful for you. The public (unregistered) version has all

the features as the registered version except the "Create report file"

feature. The other disadvantage is the message that HWiNFO isn't registered

and that you have to press a random key while the results are shown.

If you think you could use this program and don't want to see that stupid

message, or if you continue using it after a trial period of 14 days, you

should register. Otherwise after this period HWiNFO becomes an illegal copy

and you have to delete it (ho-ho-ho :-)).

If you decide to register, you should print out the file REGISTER.FRM,

fill it and send to the address in the file.

If you include your Internet e-mail address, you will receive the

registration code via e-mail, otherwise snail mail.

Once you have obtained the nice code needed to register, put exactly the

same name, and the code into the [About|Register] menu of HWiNFO and the

current version will become registered.

Well, the Registration fee is:

�������������������������������������ͻ

�  Non-Commercial  �    Commercial    �

�������������������������������������͹

� DEM � US$ � Sk/KC� DEM � US$ � Sk/KC�

�����������������������������������������������������͹

� 1 User        �  35 �  20 �  500 �  70 �  40 � 1000 �

�����������������������������������������������������Ķ

� 2-10 Users    �* 30 �* 16 �* 450 �  60 �  32 �  900 � per copy

�����������������������������������������������������Ķ

� 11-20 Users   �* 25 �* 13 �* 400 �  50 �  26 �  800 � per copy

�����������������������������������������������������Ķ

� 21-50 Users   �* 20 �* 10 �* 300 �  40 �  20 �  600 � per copy

�����������������������������������������������������Ķ

� 51-100 Users  �* 15 �*  7 �* 200 �  30 �  14 �  400 � per copy

�����������������������������������������������������Ķ

� UNLIMITED     �Contact the author�Contact the author�

�����������������������������������������������������ͼ

* for schools, educations, etc.

When registering with Eurocheque or bank transfer please add 10% to

the total price to cover bank transfer costs.

For any special requirements contact the author.

A registration is valid for all future versions.

And there will be a lot of newer versions of course.

You should register NOW !

## 6. Creating a logfile #####################################################

You can create a log-file by pressing the <F2> key; it will add the results

from current screen to a file named "hwinfo.log". You can change the name of

this file by entering the .log filename you want, as a parameter to hwinfo;

e.g. "hwinfo.exe my_p7.log".

There is also a "Create report file" feature, but it's available only for

registered users. It will create automatically a complete logfile, when

you start HWiNFO using the "-r" switch. There is no need to navigate HWiNFO

through all screens.

## 7. Limitations ############################################################

The Mainboard Chipset identification works only on newer (� 1991 and above)

AMI and newer AWARD BIOSes. HWiNFO is also able to detect some chipsets, not

depending on the type of BIOS (see HWiNFO.LST).

Fixed Disk Buffered Data Transfer rate reads data from one physical sector

until 4 seconds. Some caches (like Hyperdisk; but NCACHE or SMARTDRV don't)

could cause invalid results. Also some mainboards, which have incompatible

timers could cause problems.

Fixed Disk Average Seek receives results while seeking permanently 100* from

cylinder 0 to a cylinder, which is 1/3 of the max. cylinder number. Thus some

programs could report another result if they seek to an another cylinder.

SCSI disks and some Cache programs (Hyperdisk) don't generate Seek requests,

thus the Seek and RPM result is abnormal and HWiNFO displays <N/A>.

PCMCIA detection requires Socket Services or Card Services to be installed.

If you want more info about sockets, install Card Services.

The VESA Local Bus can be detected only if a VLB card is present.

CD-ROM benchmark requires MSCDEX.EXE (Microsoft CD-ROM Extensions) to

be installed and an inserted data CD.

## 8. Running HWiNFO under V86 Memory Managers ###############################

HWiNFO (v3.00+) is able to run in Virtual86 mode under some circumstances

(it means, under some applications, which turn the V86 mode on). But ONLY

those which support the Windows INIT BROADCAST and handle it correctly.

Such known applications are:

- Microsoft EMM386 Expanded Memory Manager

- Quarterdeck QEMM386 Expanded Memory Manager

- Qualitas 386MAX Expanded Memory Manager

- Nu-Mega Soft-ICE Debugger

.......

These applications are able to switch to Real mode when they receive the

Windows INIT BROADCAST message. But if you run HWiNFO in V86 there may

occur some problems, especially during CPU RESET (like hang, crash...).

HWiNFO works best under EMM386 (I haven't found any problems).

There are some disadvantages while runnig in V86 mode:

- The method of detecting CPU frequency via TSC (Time Stamp Counter)

doesn't work (can't execute RDTSC in V86).

- The FPU benchmark returns another results (it's slower) as in real mode.

- The V86 manager is able to relocate BIOSes, so HWiNFO won't be able

to identify BIOS type and some video cards.

- A crash while detecting the CPU type could happen on some older

CPUs requiring CPU RESET to identify them properly.

<<< So, I prefer runnig HWiNFO in Real Mode. >>>

HWiNFO is unable to run under Microsoft Windows and Windows95 DOS box,

Windows NT VDM and IBM OS/2, because they don't switch to Real Mode and

also manipulate Benchmark results, BIOS scan, are able to control direct

I/O operations, which can cause invalid results and Windows NT doesn't

allow 16-bit applications to directly access the disk and some hardware

routines cause problems, thus it can't be supported.

## 9. Resetting the CPU ######################################################

The CPU ID is available only on 386+ CPUs and HWiNFO uses three ways to

obtain this ID:

Method 1 - Resets the CPU while A20 is Enabled.

This method works on all systems, but the Reset handler gets

control after BIOS. And some BIOSes (some newer AMI/Award BIOSes)

destroy CPU ID (EDX) and therefore this ID could be invalid.

This method is default <Enabled> in Setup section.

Method 2 - Resets the CPU while A20 is Disabled.

This method works only on some machines, which do have a

Fully-Terminated bus. It's the best way to obtain CPU ID, because

it gets control immediately after Reset (the BIOS can't destroy

EDX). I've found some problems with this method while running

with Norton cache.

This method is default <Disabled> in Setup section.

Method 3 - Try to use this method if your BIOS destroys CPU ID. But it works

only on systems, which do not have Main BIOS (area at 0F000:0)

Write-Protected, because it modifies it for a bit, and then

restores back. This method is on some machines the only way

to obtain CPU ID.

This method is default <Enabled> in Setup section.

HWiNFO tries to determine CPU ID this way, via: 1. CPUID instruction

2. BIOS (INT 15h)

3. Method 2 (if Enabled)

4. Method 3

5. Method 1

If you have an UMC U5SX/SD/SLV/486DX2/SX2, Enhanced Am486DX2/DX4/DX4-WB,

AMD-K5 and above, Intel 486DX/SX/DX2 SL-Enhanced, IntelDX4, IntelSX2,

IntelDX2 W-B Enh. or Intel Pentium(R) and above, there is no need to Reset

the CPU (these CPU support CPUID instruction). Most CPUs today support

CPUID instruction.

Note: NexGen Nx586 does NOT support the CPUID instruction.

Cyrix 5x86 and above support the CPUID instruction only if CPUIDEN bit

in CCR4 register is set. HWiNFO 3.0.7+ sets this bit automatically.

Disabling "CPU ID detection via Method 1, 2 and 3" reduces your CPU

detection ability. And if there's no other way to obtain this ID

(via BIOS or CPUID instruction), it won't identify 386SL, i386CX/EX,

486DX2, IBM 486... and some other CPUs. Do this only if your system

hangs while attempting CPU detection.

Some BIOSes simply OVERWRITE the CPU ID (EDX) :-< ! In this case an

invalid CPU ID is displayed and the CPU is probably incorrectly identified.

If this problem occurs, try to use Method 3 (but your Main BIOS must be

Write-Enabled !). If it doesn't work, try to use Method 2 (but you must

have a Fully-Terminated bus !). And if HWiNFO still displays, that Method 1

was used to obtain CPU ID, not Method 2 nor 3 can obtain it.

## 10. HWiNFO Crash ##########################################################

- S3 ViRGE cards seem to crash while performing the TV Tuner test.

If so, disable the "Check TV Tuner" option in Setup.

- Stacker Compression Card and 3Com EtherLink III Adapter detection

routines can cause crashes with some Network cards. You can disable

them in the Configuration section (Stacker check is default disabled;

3Com EtherLink III detection is enabled).

- If HWiNFO crashes while starting or while the "Flushing Disk Caches..."

message is displayed, disable the "FlushDiskCache" option in HWiNFO.INI

or HWiNFO|Setup.

## 11. Secondary Video Info ##################################################

RISC Graphics Processor: currently identifies only Hitachi HD63484 ACRTC

Scanning for TMS: checks for TMS340x0 processor on several ports and tries

to get vendor and device name (detection routine from

SPEA GDC drivers, I don't know if it works with other TMS

cards too)

TMS Boot EEPROM: gets info from Boot Software; this won't probably work

with other cards than SPEA (I don't know :-( )

TIGA driver: gets info from TIGA driver if it's loaded (TIGACD.EXE)

Also found as: identifies all SPEA Graphiti series cards from GDCINI.DAT

and checks for HP IGC-20 and NEC MultiSync Graphics Engine

You can obtain info such as:

e.g.: RISC Graphics Processor:   Hitachi HD63484 (ACRTC)

Scanning for TMS:          N/A

TMS Boot EEPROM:           N/A

TIGA driver:               N/A

Also found as:             SPEA Graphiti Flash 1G2-HE

or:   RISC Graphics Processor:   N/A

Scanning for TMS:          (C) SPEA Software AG HILITE1024

TMS Boot EEPROM:           SPEA HiLite1024

TIGA driver:               N/A

Also found as:             SPEA Graphiti HiLite1024

or:   RISC Graphics Processor:   N/A

Scanning for TMS:          <I really don't know what it'll display>

TMS Boot EEPROM:                             -||-

TIGA driver:               HP A1083A Intelligent Graphics Controller 20

Also found as:             Hewlett Packard IGC-20 A1083A

## 12. PCI chipsets comparison ###############################################

������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������Ŀ

� Chipset        � Intel � Intel � Intel � Intel � Intel � Intel � Intel � Intel � Intel � Intel � Intel �  VIA  �  VIA  �  VIA  �  VIA  �  VIA  � SiS   �  SiS  � SiS   � SiS   � SiS   � SiS   �

�                � 430FX � 430VX � 430HX � 430MX � 430TX � 440FX � 450GX � 450KX � 440LX � 440EX � 440BX �  VP1  �  VPX  �  VP2  �  VP3  �  MVP3 � 5511  �  5596 � 5571  � 5581  � 5597  � 5591  �

������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������Ĵ

� North Bridge   �82437FX�82437VX�82439HX�82437MX�82379TX�82441FX�82453GX�82453KX�82443LX�82443EX�82443BX�82C585 �82C585 �82C595 �82C597 �82C598 � 5511  � 5596  � 5571  � 5581  � 5597  � 5591  �

�                �  TSC  �  TVX  �  TXC  � MTSC  � MTXC  �  PMC  �82454GX�82454KX�  PAC  �  PAC  �       �  VP   �  VPX  �       �  /AT  �  /AT  � PCMC  �       �       �       �       �       �

������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������Ĵ

� Supported      � P54C  � P54C  � P54C  � P54LM � P54C, � P6,   � P6    � P6    � P-II  � P-II  � P-II  � P54C, � P54C, � P54C, � P54C, � P54C, � P54C, � P54C, � P54C, � P54C, � P54C, � P54C, �

� CPUs           � K5,   � K5,   � K5,   �mobile � P55C, � P-II  �       �       �       �Celeron�       � K5,   � P55C, � P55C, � P55C, � P55C, � K5,   � K5,   � P55C, � P55C, � P55C, � P55C, �

�                � M1    � M1    � M1    �       � K5,K6 �       �       �       �       �       �       � M1    � K5,K6 � K5,K6 � K5,K6 � K5,K6 � M1    � M1    � K5,K6,� K5,K6,� K5,K6,� K5,K6,�

�                �       �       �       �       � M1,M2 �       �       �       �       �       �       �       � M1,M2 � M1,M2 � M1,M2 � M1,M2 �       �       � M1,M2 � M1,M2 � M1,M2 � M1,M2 �

������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������Ĵ

� Max. bus freq. �  66   �  66   �  66   �  66   �  75   �  66   �  66   �  66   �  66   �  66   �  100  �  66   �  75   �  66   �  100  �  100  �  66   �  66   �  75   �  75   �  75   �  66   �

������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������Ĵ

� SMP (max CPUs) �   -   �   -   �   2   �   -   �   -   �   2   �   4   �   2   �   2   �   -   �   2   �   -   �   -   �   -   �   -   �   -   �   -   �   -   �   -   �   -   �   -   �   -   �

������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������Ĵ

� Max. memory    � 128M  � 128M  � 512M  � 128M  � 256M  �  1G   �  8G   �  1G   �1G(EDO)� 256M  �  1G   � 512M  � 512M  � 768M  �  1G   �  1G   � 512M  � 512M  � 384M  � 384M  � 384M  � 768M  �

������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������Ĵ

� Supported      �SPM,EDO�FPM,EDO�FPM,EDO�SPM,EDO�FPM,EDO�FPM,EDO�  FPM  �  FPM  � EDO,  � EDO,  � EDO,  � FPM,  � FPM,  � FPM,  �FPM,EDO�FPM,EDO�FPM,EDO�FPM,EDO� FPM,  � FPM,  � FPM,  � FPM,  �

� Memory         �       � SDRAM �       �       � SDRAM � BEDO  �       �       � SDRAM � SDRAM � SDRAM � EDO,  � EDO,  � EDO,  �SDRAM  �SDRAM  �       �       � EDO,  � EDO,  � EDO,  � EDO,  �

� Types          �       �       �       �       �       �       �       �       �       �       �   **  � SDRAM � SDRAM � SDRAM �SDRAM2 �SDRAM2 �       �       � SDRAM � SDRAM � SDRAM � SDRAM �

������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������Ĵ

� ECC/Parity     �   -   �   -   �   X   �   -   �   -   �   X   �   X   �   X   �   X   �   -   �   X   �   -   �   -   �   X   �   X   �   X   �   -   �   -   �   -   �   -   �   -   �   -   �

������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������Ĵ

� Cacheability   �  64M  �  64M  �  512M �  64M  �  64M  �  ***  �  ***  �  ***  �  ***  �  ***  �  ***  � 512M* � 512M* � 512M* � 512M *� 512M *� 256M *� 256M *� 128M *� 128M *� 128M *� 256M *�

������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������Ĵ

� South Bridge   �82371FB�82371SB�82371SB�82371MX�82371AB�82371SB�   -   �   -   �82371AB�82371AB�82371EB�82C586 �82C586B�82C586B�82C586B�82C586B� 5513  � 5513  � 5513  � 5513  � 5513  � 5595  �

�                � PIIX  � PIIX3 � PIIX3 � MPIIX � PIIX4 � PIIX3 �       �       � PIIX4 � PIIX4 �PIIX4/E�       �       �       �       �       �       �       �integr.�integr.�integr.�       �

������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������Ĵ

� PCI support    �  2.0  �  2.1  �  2.1  �  2.0  �  2.1  �  2.1  �  2.0  �  2.0  �  2.1  �  2.1  �  2.1  �  2.1  �  2.1  �  2.1  �  2.1  �  2.1  �  2.0  �  2.0  �  2.1  �  2.1  �  2.1  �  2.1  �

������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������Ĵ

� A.G.P.         �   -   �   -   �   -   �   -   �   -   �   -   �   -   �   -   �  2x   �  2x   �  2x   �   -   �   -   �   -   �  2x   �  2x   �   -   �   -   �   -   �   -   �   -   �   X   �

������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������Ĵ

� Concurrent PCI �   -   �   X   �   X   �   -   �   X   �   X   �  ***  �  ***  �   X   �   X   �   X   �   X   �   X   �   X   �   X   �   X   �   X   �   X   �   X   �   X   �   X   �   X   �

������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������Ĵ

� USB            �   -   �   X   �   X   �   -   �   X   �   X   �  ***  �  ***  �   X   �   X   �   X   �   X   �   X   �   X   �   X   �   X   �   -   �   -   �   X   �   X   �   X   �   X   �

������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������Ĵ

� Power Mgmt.    �   -   �   -   �   -   �SMI/APM� ACPI  �  SMM  �  SMM  �  SMM  � ACPI  � ACPI  � ACPI  �  APM  �  ACPI �  ACPI � ACPI  � ACPI  �  SMM  �  SMM  �  SMM  � ACPI  � ACPI  � ACPI  �

������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������Ĵ

� Ultra DMA/33   �   -   �   -   �   -   �   -   �   X   �   -   �   -   �   -   �   X   �   X   �   X   �   -   �   X   �   X   �   X   �   X   �   -   �   -   �   -   �   X   �   X   �   X   �

��������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������

*    - Depends on L2 cache size:

�����������������������Ŀ

� cache �   cacheable   �

� size  �    region     �

�����������������������Ĵ

� 256K  �64M-cache_size �

� 512K  �128M-cache_size�

� 1M    �256M-cache_size�

� 2M    �512M-cache_size�

�������������������������

**   - EDO and SDRAM cannot be mixed

***  - no L2 cache on-board (depends on on-chip L2 cache size in CPU)

**** - available if 82371SB PIIX3 is used

## 13. IDE Disk Transfer Modes ###############################################

HWiNFO reports transfer modes for all IDE/ATAPI drives, which support it.

Max. PIO transfer mode:   PIO x

DMA Transfer Modes:       SW: a1 (m1), MW: a2 (m2), Ultra: a3 (m3)

Where:

x = Maximum supported PIO transfer mode

SW = Singleword DMA transfer mode

MW = Multiword DMA transfer mode

Ultra = Ultra-DMA transfer mode

a1, a2, a3 = Current active mode

m1, m2, m3 = Max. supported mode

�����������������������������Ŀ

� Transfer    � Maximum       �

� Mode        � Transfer rate �

�����������������������������Ĵ

� PIO 0       �    3.33 MB/s  �

� PIO 1       �    5.22 MB/s  �

� PIO 2       �    8.33 MB/s  �

� PIO 3       �   11.11 MB/s  �

� PIO 4       �   16.66 MB/s  �

�����������������������������Ĵ

� DMA 0       �    4.16 MB/s  �

� DMA 1       �   13.33 MB/s  �

� DMA 2       �   16.66 MB/s  �

�����������������������������Ĵ

� Ultra-DMA 0 �   16.66 MB/s  �

� Ultra-DMA 1 �   25.00 MB/s  �

� Ultra-DMA 2 �   33.33 MB/s  � Ultra-DMA/33

�������������������������������

## 14. Miscellaneous notes ###################################################

On some old motherboards, random numbers are obtained from unused I/O ports

and this may cause invalid identification of some hardware (e.g. WSS).

I don't know how to solve this problem.

Video Throughput benchmark depends only on External CPU frequency and the

type of Video board used; e.g. 486DX-33 with the same graphics board as an

i486DX4-100 (running at 3*33 MHz) report the same Video Throughput speed !

Intel Pentium(R) and above CPUs do not report, that the "APIC on-chip"

feature is present. Most of them (P54C yes, but P54CQS and P54LM don't)

have an on-chip APIC, but don't report it, if it isn't supported by

external hardware.

Dhrystone benchmark tests the performance of CPU and Memory, but only in

16-bit code. Therefore a 150 MHz Pentium Pro (which is designed and tuned

especially for 32-bit code) returns a rather low Dhrystone result.

So its performance while running with 16-bit applications is comparable

to a 100 MHz Pentium P54C.

If you have an Intel 420LX/420NX/430FX/430HX/430VX/430MX/430TX,

SiS 85C501/5120/5511/5571/5581/5596/5597/5591, AMD-640 chipset or

VIA Apollo VP-1, VPX, VP3, HWiNFO is able to detect the type of

External Cache and its size direct from the PCI chipset registers.

## 15. Some abbreviations ####################################################

FPM          = Fast-Page Mode (old standard DRAM type)

EDO          = Extended Data-Out

SDRAM        = Synchronous DRAM

DDR SDRAM II = Double Data Rate SDRAM II

BEDO         = Burst EDO

EC           = Error Check only

ECC          = Error Check and Correction

DRAM         = Dynamic RAM

VRAM         = Video RAM

WRAM         = Window RAM

SDRAM        = Synchronous DRAM

SGRAM        = Synchronous Graphics RAM

RDRAM        = Rambus DRAM

MDRAM        = Multibank DRAM

VRT          = Voltage Reduction Technology

APIC         = Advanced Programmable Interrupt Controller (required for DP)

DP           = Dual Processing

FRC          = Functional Redundancy Checking (requires DP)

MMX          = Multimedia Extensions

(57 new instructions, SIMD architecture, 64-bit regs)

S.E.C        = Single Edge Contact

(CPU and Level-2 cache on one substrate)

TLB          = Translation Lookaside Buffer

(small cache for page translation)

PnP          = Plug-and-Play

ESCD         = Extended System Configuration Data

DMI          = Desktop Management Interface

ACPI         = Advanced Configuration and Power Interface

APM          = Advanced Power Management

VESA/VBE     = Video Electronics Standards Association/Video BIOS Extensions

DDC          = Display Data Channel

(DDC1: Monitor->PC, DDC2B: Monitor<->PC)

## 16. Codenames #############################################################

Codenames for Intel Processors:

P5          - Intel Pentium(R) Processor:

Old 50/50 (Engineering Sample), 60/60 and 66/66 MHz.

P54C        - Intel Pentium(R) Processor:

Later 75/50, 90/60, 100/50, 100/66, 120/60, 133/66, 150/60,

166/66, 180/60, 200/66 and 233/66 MHz without VRT, with

on-chip APIC, Dual Processing Features and FRC support.

P54CM       - Intel Pentium(R) Update Processor:

Old 90MHz Slave processor in Dual Processor system.

P54CQS      - Intel Pentium(R) Processor:

75/50, 100/66, 120/60, 133/66, 166/66 and 200/66 MHz

without VRT, on-chip APIC, Dual Processing Features, Upgrade

and FRC support.

P54LM       - Intel Pentium(R) Mobile Processor:

75/50, 120/60, 133/66 and 150/60 Low Power versions (2.9 V)

supporting VRT. These Processors are shipped without APIC

on-chip, Dual Processing Features, Upgrade and FRC support.

P24T        - Intel Pentium(R) OverDrive(R) Processor:

PODP5V63 63/25 MHz and PODP5V83 83/33 MHz OverDrive

for 486 CPUs.

P24CT       - Intel Pentium(R) OverDrive(R) Processor:

OverDrive for IntelDX4(TM) Processor.

P5T         - Intel Pentium(R) OverDrive(R) Processor:

PODP5V120 120/60 MHz and PODP5V133 133/66 MHz OverDrive for

Intel Pentium(R) P5 60/60 and 66/66 MHz Processors.

P54CT       - Intel Pentium(R) OverDrive Processor:

PODP3V125 125/50 MHz, PODP3V150 150/60 MHz and

PODP3V166 166/66 MHz OverDrive for Intel Pentium(R) P54C

75/50, 90/60 and 100/66 MHz Processors.

P55C        - Intel Pentium(R) Processor with MMX(TM) Technology:

166/66, 200/66, 233/66 and 266/66 MHz

MMX, 32 KBytes First Level Cache and better Branch Prediction.

P55LM

Tilamook    - Mobile Intel Pentium(R) Processor with MMX(TM) Technology:

200/66 and 233/66 MHz

P55CTP      - Intel Pentium(R) OverDrive(R) Processor with MMX(TM) Technology:

PODPMT60X150 125/50, 150/60, PODPMT66X166 166/66,

PODPMT60X180 180/60 and PODPMT66X200 200/66 for Intel Pentium(R)

75/50, 90/60 and 100/66 Processors.

P6          - Intel Pentium(R) Pro Processor:

133/66 (Engineering Sample), 150/60, 166/66, 180/60 and 200/66,

256K, 512K and 1024K of L2 cache.

P6T         - Intel Pentium(R) Pro OverDrive(R) Processor.

P6L

Klamath     - Intel Pentium(R) II Processor:

233/66, 266/66, 300/66, 333/66,

P6 with MMX(TM) Technology and without Level-2 on-chip cache,

S.E.C. cartridge, Slot 1, 512K cache on substrate,

VccCORE=2.8V, up to 2 CPUs in MPS

Celeron     - Intel Celeron(TM) Processor:

266/66 MHz, no L2 cache, MMX2

???         - Mobile Intel Pentium(R) II Processor:

233/66 and 266/66 MHz, VccCORE=1.7V

Deschutes   - Intel Pentium(R) II Processor:

350/100 and 400/100 MHz, 512K L2 cache, MMX2, VccCORE=2.0V

Xeon        - Intel Pentium(R) II Xeon Processor:

400/100 MHz, 512K, 1M or 2M L2 cache, MMX2, Slot 2,

up to 8 CPUs in MPS

Mendocino   - 300 MHz, 128K L2 on-chip cache

Katmai      - MMX2, 100 MHz bus clock

Williamette - Enhanced Pentium Pro

Merced      - Intel + HP project

IA64 architecture, VLIW

## 17. Thanks ################################################################

I want especially to thank:

� Beta testers: Boris Hur�nek [BHsoft] (hurinek@axa.sk)

Erik Francisci (jumper!franci@dec3.dcs.elf.stuba.sk)

Igor Junas (junas@axis.sk)

Mat�s Rosa (rosa@???.sk)

Milan Marusinec (marusin@lithec.sk)

� Alex V. Potemkin and Potemkin's Hackers Group (avp@stelth.dvm.msk.ru)

� Aad Offerman (offerman@einstein.et.tudelft.nl) - CHIPLIST

� J�n G�cik (jan.gucik@vslib.cz) - IntroScope

� Tom�s Lehuta (lehuta@elf.stuba.sk) - GUS Memory routs, hints

� Peter Hubinsky (hubak@elf.stuba.sk; SAC fileadmin) - distribution

� Emil Laurentiu (lcarmen@u1.ici.ro) - hints

� Andrew Nelson (anelson@atlas.co.uk) - English language suggestions :-)

� Marek Smihla (ceit@internet.sk) - reports & hints

� Jiang Hong (zlzhang@pku.edu.cn) - BL3 & hints

� Robert Cline, Tim McCarthy (support@intel.com) - id of Intel CPUs

� Tamas Miklos (fieryofup@geocities.com) - WSS&Modem rout, hints, ASMDEMO

� ESCOM CS (escom_support@brn.pvtnet.cz) - info

� Olivier Cahagne (cahagn_o@epita.fr) - SysInfo observatory

� Thomas Mainka (100112.2132@compuserve.com) - reports

� Christian Klak (drk-soft@gmx.net) - Acer info

� Gershon Mikulsky (mikuls@shani.net) - ATI info

� Janusz Bubik (j.bubik@micronic.cz) - reports

� All other people, who have sent bug reports and suggestions (*!*@*.*)

Greetings to:

� Lubos Vrtik (vrtik@vuje.sk) - cracking <-> protecting impressions, HMVS

� C00LMAN of Siam Hacker's Gang - KeY GeNeRaToR 4 HWiNFO v<4.0.0

(I'm waiting 4 keYGen 4 HWiNFO v4.0.0+)

� eGIS / pCE - *v3Ry nICE* K3YG3N 4 HWiNFO �4.2.4, 4.2.5

(No idea how to receive the true magic numbers for v4.0.0+ ?)

� dEVIL / uCF - *nICE* kEYZ 4 HWiNFO v4.2.2, ReGGeD v4.3.0

(try brute force attack ;-))

## 18. Disclaimer ############################################################

����������������������������������������������������������������������������ͻ

�  All product names are copyright and registered trademarks/tradenames of   �

�                        their respective owners.                            �

����������������������������������������������������������������������������ͼ

HWiNFO IS COPYRIGHTED MATERIAL AND PROTECTED BY APPLICABLE

COPYRIGHT LAWS AND INTERNATIONAL TREATY PROVISIONS.

THE PROGRAM AND ITS DOCUMENTATION ARE COPYRIGHTED BY THE ORIGINAL

PROGRAMMER/AUTHOR (COPYRIGHT HOLDER). ALL RIGHTS ARE RESERVED CONCERNING

FUTURE CHANGES OF THE NAME AND CONTENTS OF THIS DISTRIBUTION PACKAGE. NO

CHANGES MUST BE MADE FOR ANY REASONS AT ANY TIME BY ANYBODY IN ANY ASPECT,

INCLUDING, BUT NOT LIMITED TO, MODIFICATION OF THE COPYRIGHT NOTICE OR

CHANGES OF THE COMPILED BINARIES HEREBY ALTERING THE ORIGINALLY INTENDED

PROGRAM EXECUTION FLOW.

RealSoft SPECIFICALLY DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, AND

ALL LIABILITY, INCLUDING CONSEQUENTIAL AND OTHER INDIRECT DAMAGES, FOR

THE USE OF THIS SOFTWARE, INCLUDING LIABILITY FOR INFRINGEMENT OF ANY

PROPRIETARY RIGHTS. RealSoft DOES NOT ASSUME ANY RESPONSIBILITY FOR ANY

ERRORS WHICH MAY APPEAR IN THIS CODE NOR ANY RESPONSIBILITY TO UPDATE IT.

THE PERSON USING THE SOFTWARE BEARS ALL RISKS AS TO THE QUALITY AND

PERFORMANCE OF THIS SOFTWARE.

## END #######################################################################