The Class 2 Command Sets

������������������������

THE CLASS 2 COMMAND SET

The following class 2 commands are supported and implemented per EIA PN2388

(8/20/90).

Command Style

+<command>=<value>	Execute a command or set a parameter

+<command>?	Read possible settings

+<command>=?	Read current setting

Supported Commands (per EIA PN2388 8/20/90)

+FAA=n		Auto answer mode parameter

n=0	DCE to answer as set by +FCLASS

n=1	DCE can answer and auto. determine calling type

+FCLASS=n		Service class selection

n=0	Set to modem mode

n=2	Set to Class 2 mode

n=6	Set to ZFAX mode

n=8	Set to Voice mode

+FBOR=n 	Phase C data bit order

n=0	Select direct bit order

n=1	Select reversed bit order in receiving mode

+FCIG="string"          Local fax station ID string, for polling Rx.

+FCON		DCE response, Fax connection made

+FCR=n		Capability to receive parameter

n=0	DCE will not receive message data or poll a remote

+FDCC=vr,br,wd,ln,df,ec,bf,st	DCE capabilities parameters

vr=0 Vertical resolution ,Normal 98 lpi

vr=1 Vertical resolution ,Fine 196 lpi

br=0 Bit rate ,2400 bit/s V.27ter

br=1 Bit rate ,4800 bit/s V.27ter

br=2 Bit rate ,7200 bit/s V.29 or V.17

br=3 Bit rate ,9600 bit/s V.29 or V.17

br=4 Bit rate ,12000 bit/s V.17

br=5 Bit rate ,14400 bit/s V.17

wd=0 Page Width,1728 pixels in 215mm

wd=1 Page Width,2048 pixels in 255mm

wd=2 Page Width,2432 pixels in 303mm

ln=0 Page length,A4 297mm

ln=1 page length,B4 364mm

ln=2 page length,unlimited length

df=0 Data compression format,1-D modified Huffman

df=1 Data compression format,2-D modified Read

ec=0 Error correction disabled

bf=0 disable Binary file transfer

st=0 minimum scan time/line 0 ms

st=1 minimum scan time/line 5 ms

st=2 10 ms (normal) , 5 ms (fine)

st=3 minimum scan time/line 10ms

st=4 20 ms (normal) , 10ms (fine)

st=5 minimum scan time/line 20ms

st=6 40 ms (normal) , 20ms (fine)

st=7 minimum scan time/line 40ms

+FDCS=vr,br,wd,ln,df,ec,bf,st	Current session parameter; refer to +FDCC command

+FDIS=vr,br,wd,ln,df,ec,bf,st	Current session negotiation parameter; refer to +FDCC command

+FDR		Receive phase C data command; initiates document reception

+FDT=df,vr,wd,in		Transmit phase C data command; releases the DCE to proceed with negotiation

+FET=n		End the page or document command

n=0	More pages; same document

n=1	End of document; another document follows

n=2	No more pages or documents

n=4	Procedure interrupt: another page follows

n=5	Procedure interrupt: end of document, another document follows

n=6	Procedure interrupt: end of document

+FLID="string"          Local ID string parameter

+FLPL=n 	Document for polling command

n=0	Indicate that the DTE has no document for polling

n=1	Indicate document available for polling

+FMDL?		Request DCE model

+FMFR?		Request DCE manufacturer

+FPTS=n 	Page transfer status

n=1	Received page good

n=2	Page bad; retrain requested

n=3	Page good; retrain requested

n=4	Page bad; procedure interrupt requested

n=5	Page good; procedure interrupt requested

+FNS="<string of Hex>"  Non-standard frame FIF octet string

+FREV?		Request DCE revision

+FSPL=n 	Enable polling command

n=0	Disable polling

n=1	Enable polling

All other +F commands are not supported but the modem will respond OK.  In

many cases this means "don't care".  See PN 2388 for command details.

The Class 2 Responses

���������������������

RESPONSE			FUNCTION AND DESCRIPTION

���������			�������������������������

+FCIG:"String"                  Report remote ID response CIG

+FCFR				Confirmation to receive prompt

+FCON				Facsimile connection response

+FCSI:"String"                  Report remote ID response CSI

+FDCS:vr,br,wd,ln,df,ec,bf,st	Report session parameters response ,refer to

+FDCC=.... command

+FDIS:vr,br,wd,ln,df,ec,bf,st	Report remote capabilities response,refer to

+FDCC=.... command

+FDTC:vr,br,wd,ln,df,ec,bf,st	Report remote capabilities response ,refer to

+FDCC=.... command

+FET:n				Post page message response,refer to +FET=n command

+FHNG:nnn			Call termination status response

nnn=00	Normal and proper end of connection

nnn=10	Transmit error on phase A hangup code

nnn=20	Transmit error on phase B hangup code

nnn=40	Transmit error on phase C hangup code

nnn=50	Transmit error on phase D hangup code

nnn=70	Receive error on phase B hangup code

nnn=90	Receive error on phase C hangup code

nnn=100 Receive error on phase D hangup code

+FNSC:"HEX string"              Report the non-standard facilities command frame

For example the polling security ID

+FNSF:"HEX string"              Report the non-standard facilities frame response

+FPOLL				Remote polling indication

+FPTS:n 			Receive page transfer status response,refer to

+FPTS=n command

+FTSI:"String"                  Report remote ID response TSI

+FVOICE 			Transition to Voice response.

Class 2 Fax Examples

���������������������

Legend:

1.	"------>"    :Command from DTE(computer) to DCE(ZyXEL series modem)

2.	"<------"    :Response from DCE(ZyXEL series modem) to DTE(computer)

3.	"<XON>"      :ASCII <DC1> character ,049 decimal ,031 hexdecimal

4.	"<DLE>"      :ASCII <DLE> character ,016 decimal ,010 hexdecimal

5.	"<ETX>"      :ASCII <ETX> character ,003 decimal

6.	"<DC2>"      :ASCII <DC2> character ,050 decimal ,032 hexdecimal

7.	"<XOFF>"     :ASCII <DC3> character ,051 decimal ,033 hexdecimal

8.	"(T.4 DATA)" :Refer to CCITT blue book volume VII T.4

Example 1: Send 2 pages FAX message  (refer to Example 2)

Direction   Command or response        description

���������   �������������������        �������������

------>     AT&F		       ;Initinalze Modem to Default setting.

<------     OK

------>     AT+FCLASS=2 	       ;after any AT command the baud

rate will detected automatically

<------     OK

------>     AT+FLID=886 35 783734      ;Local ID string

<------     OK

------>     AT+FDCC=1		       ;set to fine resolution mode

<------     OK

------>     ATDT027718546	       ;Dial out the number

<------     +FCON		       ;Fax connection response

<------     +FNSF:00 00 00 00

<------     +FCSI:999 2 7718546        ;report the remote fax identification

<------     +FDIS:1,5,0,2,0,0,0,0      ;report the remote fax machine capability

<------     OK

------>     AT+FDT

<------     +FDCS:1,5,0,2,0,0,0,0      ;report the current session parameter

<------     CONNECT

<------     <XON>

------>     (T.4 DATA)		       ;T.4 data,need to take care of flow control

------>     <DLE> <ETX> 	       ;return to control

<------     OK

------>     AT+FET=0		       ;End of page more to follow.

<------     +FPTS:1

<------     OK

------>     AT+FDT

<------     CONNECT

<------     <XON>

------>     (T.4 DATA)		       ;T.4 data,need to take care of flow control

------>     <DLE> <ETX> 	       ;return to control

<------     OK

------>     AT+FET=2		       ;End of document and session.

<------     +FPTS:1

<------     +FHNG:00		       ;Fax transmition completed with no error

<------     OK

<------     NO CARRIER		       ;Modem response,indicate line is terminate

Example 2: Received 2 pages FAX message  (refer to Example 1)

Direction   Command or response        description

���������   �������������������        �������������

------>     AT&F		       ;Initinalze Modem to Default setting.

<------     OK

------>     AT+FCLASS=2 	       ;set Modem to Class 2 fax mode

<------     OK

------>     AT+FLID=999 2 7718546      ;local ID string

<------     OK

------>     AT+FCR=1		       ;Enable FAX receive

<------     OK

------>     AT+FDCC=1		       ;set to fine resolution

<------     OK

<------     RING		       ;Remote FAX machine Dial in

<------     RING

------>     ATA 		       ;Computer answer the call

<------     +FCON		       ;Fax connection response

<------     +FTSI:886 35 783734        ;report the remote fax identification

<------     +FDCS:1,5,0,2,0,0,0,0      ;report the remote fax machine capability

<------     OK

------>     AT+FDR

<------     +FCFR		       ;confirm for FAX receive

<------     +FTSI:886 35 783734

<------     +FDCS:1,5,0,2,0,0,0,0      ;report the remote fax machine capability

<------     CONNECT

------>     <DC2>

<------     (T.4 DATA)		       ;T.4 data,need to take care of flow control

<------     <DLE> <ETX>

<------     +FPTS:1		       ;Page good

<------     +FET:0		       ;more pages follow

<------     OK

------>     AT+FDR

<------     CONNECT

------>     <DC2>

<------     (T.4 DATA)		       ;T.4 data,need to take care of flow control

<------     <DLE> <ETX> 	       ;return to control

<------     +FPTS:1		       ;Page good

<------     +FET:2		       ;no more pages or document

<------     OK

------>     AT+FDR

<------     +FHNG:000		       ;Fax receiving completed with no error

<------     OK

Example 3: Send 1 page then Polling Receive 2 pages (refer to Example 4)

Direction   Command or response        description

���������   �������������������        �������������

------>     AT&F		       ;Initinalze Modem to Default setting.

<------     OK

------>     AT+FCLASS=2

<------     OK

------>     AT+FLID=035782439

<------     OK

------>     AT+FCR=1

<------     OK

------>     AT+FBOR=0

<------     OK

------>     AT+FDCC=1,5

<------     OK

------>     AT+FSPL=1		       ;enable polling

<------     OK

------>     AT+FCIG=001 886 35 782439  ;local FAX ID ,for polling

<------     OK

------>     at+fclass?		       ;make sure is class 2 mode?if necessary

<------     2

<------     OK

------>     ATDT783734

<------     +FCON

<------     +FNSF:00 00 00 00

<------     +FCSI:035783734

<------     +FDIS:1,5,0,2,1,0,0,0

<------     +FPOLL		       ;report the remote FAX has document for

<------     OK			       ;polling

------>     AT+FDT

<------     +FDCS:0,5,0,2,0,0,0,0

<------     CONNECT

<------     <XON>

------>     (T.4 DATA)

------>     <DLE> <ETX>

<------     OK

------>     AT+FET=1

<------     +FPTS:1

<------     OK

------>     AT+FDR

<------     +FCFR

<------     +FTSI:035783734

<------     +FDCS:0,5,0,2,0,0,0,0

<------     CONNECT

------>     <DC2>

<------     (T.4 DATA)

<------     <DLE> <ETX>

<------     +FPTS:1

<------     +FET:0

<------     OK

------>     AT+FDR

<------     CONNECT

------>     <DC2>

<------     (T.4 DATA)

<------     <DLE> <ETX>

<------     +FPTS:1

<------     +FET:2

<------     OK

------>     AT+FDR

<------     +FHNG:000

<------     OK

Example 4: Receive 1 page then Polling transmite 2 pages (refer to Example 3)

Direction   Command or response        description

���������   �������������������        �������������

------>     AT&F		       ;Initinalze Modem to Default setting.

<------     OK

------>     AT+FCLASS=2

<------     OK

------>     AT+FLID=035783734

<------     OK

------>     AT+FCR=1

<------     OK

------>     AT+FBOR=0

<------     OK

------>     AT+FDCC=1,,,,1

<------     OK

------>     AT+FLPL=1		       ;Document for polling command

<------     OK

<------     RING		       ;Remote FAX machine Dial in

------>     ATA

<------     +FCON

<------     +FTSI:035782439

<------     +FDCS:1,5,0,2,0,0,0,0

<------     OK

------>     AT+FDR

<------     +FCFR

<------     +FTSI:035782439

<------     +FDCS:1,5,0,2,0,0,0,0

<------     CONNECT

------>     <DC2>

<------     (T.4 DATA)

<------     <DLE> <ETX>

<------     +FPTS:1

<------     +FET:1

<------     OK

------>     AT+FDR

<------     +FNSC:AD 00 12 00 00 50 4F 4C 4C 2D 49 44 3A 31 32 33 34 20

<------     +FCIG:001 886 35 782439

<------     +FDTC:1,5,0,2,0,0,0,0

<------     OK

------>     AT+FDT

<------     +FDCS:1,5,0,2,0,0,0,0

<------     CONNECT

<------     <XON>

------>     (T.4 DATA)

------>     <DLE> <ETX>

<------     OK

------>     AT+FET=0

<------     +FPTS:1

<------     OK

------>     AT+FDT

<------     CONNECT

<------     <XON>

------>     (T.4 DATA)

------>     <DLE> <ETX>

<------     OK

------>     AT+FET=2

<------     +FPTS:1

<------     +FHNG:00

<------     OK

Example 5: Polling receive 2 pages (refer to Example 6)

Direction   Command or response        description

���������   �������������������        �������������

------>     AT&F		       ;Initinalze Modem to Default setting.

<------     OK

------>     AT+FCLASS=2

<------     OK

------>     AT+FLID=886-35-782439

<------     OK

------>     AT+FCR=1

<------     OK

------>     AT+FDCC=1

<------     OK

------>     AT+FSPL=1

<------     OK

------>     AT+FNS=""                  ;reset to null string

<------     OK

------>     AT+FNS="AD00120000504F4C4C2D49443A35363738"  ;Polling security ID

<------     OK

------>     AT+FCIG=9-0-886-35-782439

<------     OK

------>     ATDT787045

<------     +FCON

<------     +FNSF:00 00 00 00

<------     +FCSI:011-886-35-787045

<------     +FDIS:1,5,0,2,0,0,0,0

<------     +FPOLL

<------     OK

------>     AT+FDR

<------     +FCFR

<------     +FTSI:011-886-35-787045

<------     +FDCS:1,5,0,2,0,0,0,0

<------     CONNECT

------>     <DC2>

<------     (T.4 DATA)

<------     <DLE> <ETX>

<------     +FPTS:1

<------     +FET:0

<------     OK

------>     AT+FDR

<------     CONNECT

------>     <DC2>

<------     (T.4 DATA)

<------     <DLE> <ETX>

<------     +FPTS:1

<------     +FET:2

<------     OK

------>     AT+FDR

<------     +FHNG:000

<------     OK

Example     6: Polling transmite 2 pages (refer to Example 5)

Direction   Command or response        description

���������   �������������������        �������������

------>     AT&F		       ;Initinalze Modem to Default setting.

<------     OK

------>     AT+FCLASS=2

<------     OK

------>     AT+FLID=011-886-35-787045

<------     OK

------>     AT+FDCC=1

<------     OK

------>     AT+FLPL=1

<------     OK

<------     RING			;Remote FAX machine Dial in

------>     ATA

<------     +FCON

<------     +FNSC:AD 00 12 00 00 50 4F 4C 4C 2D 49 44 3A 35 36 37 38 20

<------     +FCIG:9-0-886-35-782439

<------     +FDTC:1,5,0,2,0,0,0,0

<------     OK

------>     AT+FDT

<------     +FDCS:1,5,0,2,0,0,0,0

<------     CONNECT

<------     <XON>

------>     (T.4 DATA)

------>     <DLE> <ETX>

<------     OK

------>     AT+FET=0

<------     +FPTS:1

<------     OK

------>     AT+FDT

<------     CONNECT

<------     <XON>

------>     (T.4 DATA)

------>     <DLE> <ETX>

<------     OK

------>     AT+FET=2

<------     +FPTS:1

<------     +FHNG:00

<------     OK

Example 7: transmite FAX with procedure interrupt

Direction   Command or response        description

���������   �������������������        �������������

------>     AT&F		       ;Initinalze Modem to Default setting.

<------     OK

------>     AT+FCLASS=2

<------     OK

------>     ATDT782439

<------     +FCON

<------     +FNSF:00 00 00 00

<------     +FCSI:035783734

<------     +FDIS:1,5,0,2,0,0,0,0

<------     OK

------>     AT+FDT

<------     +FDCS:1,5,0,2,0,0,0,0

<------     CONNECT

<------     <XON>

------>     (T.4 DATA)

------>     <DLE> <ETX>

<------     OK

------>     AT+FET=4		       ;procedure interrupt ,more page

<------     +FPTS:5		       ;page good,procedure interrupt response

<------     +FVOICE		       ;response indicate transfer to phone.

<------     OK

<------     NO CARRIER

;operator intervension .(talk by phone)

------>     ATD

<------     +FCON

<------     +FNSF:00 00 00 00

<------     +FCSI:035783734

<------     +FDIS:1,5,0,2,0,0,0,0

<------     OK

------>     AT+FDT

<------     +FDCS:1,5,0,2,0,0,0,0

<------     CONNECT

<------     <XON>

------>     (T.4 DATA)

------>     <DLE> <ETX>

<------     OK

------>     AT+FET=2

<------     +FPTS:1

<------     +FHNG:00

<------     OK

FLOW CONTROL

�������������

Flow control is necessary to match the DTE-DCE date rate to the line

signalling rate while Group 3 (T.4) data is in transmission or receiving.

In Class 2 FAX mode,both hardware flow control (RTS/CTS) and software flow

control (XON/XOFF) are supported.The hardware flow control is default setting.