AMI BOARD          MAX MEM WITHOUT CARD            MAX MEM WITH CARD

MARK II XT			0 K         	                 8 MEG

MARK II AT			20 MEG	                        28 MEG

386 SX			16 MEG	                         N/A

MARK III			32 MEG	                        32 MEG

MARK IV  Screamer		64 MEG	                         N/A

MARK IV  BABY Screamer	32 MEG	                         N/A

VOYAGER			32 MEG 	                        48 MEG

VOYAGER II			32 MEG	                        32 MEG

BABY VOYAGER		32 MEG	                         N/A

ENTERPRISE  		32 MEG                          96 MEG

ENTERPRISE II		32 MEG   	   	        96 MEG

Typical System Memory Map:

0000 to 9FFF = DOS 640K memory

A000 to BFFF = Video Memory

C000 to C7FF = Video Shadowing

C800 to CFFF = SCSI, ESDI, or RLL controller

D000 to DFFF = Usually free

E000 to EFFF = Copy of main BIOS initially, may be reclaimed after boot

F000 to FFFF = Main BIOS Shadowing

RAM - Random Access Memory, Types of:

INTERLEAVED memory increases processing speed by dividing the memory

into two or four portions that process information alternatley. The CPU

sends information to one section for processing while another section

goes through a refresh cycle.

PAGE-MODE memory allows back to back memory accesses within blocks

of memory called PAGES, thereby avoiding wait states.

ROW/COLUMN is the traditonal method of accessing data at a memory

address. In this system, RAM is mapped as a matrix and a particular

address is given using a row and a column number.

In STATIC-COLUMN memory, once data is located at a specific column

address, subsequent pieces of data are accessed or allocated to the same

column. Because the column is 'static', data retrieval is faster.

SECTIONS OF MEMORY -

CONVENTIONAL memory is the first 640K, sometimes called BASE memory.

LOWER memory is the first few hundred bytes which is used by the

operating system and installable device drivers. The area left over is

used for applications and data.

UPPER memory is the area between 640K and 1Meg. It is 384K in size.

Some sections are normally reserved for use by hardware, e.g. video BIOS

(see the above typical memory map). Parts of upper memory may be used by

memory management software which can reclaim the unused portions.

Some chipsets (such as Chips & Technologies) will always reserve this

384K area for shadowing. This memory will not appear in the initial

memory count on power-up, the system configuration screen, or when using

the MEM command. This is because it is totally reserved. Other chipsets

allow a Relocation option which will re-address this memory above 1Meg as

extended memory.

EXTENDED memory is the memory located above 1Meg. AT class computers

normally run in real mode. A system must be put into Protected Mode. The

computer will still appear to an application to be running in real mode

but the features of protected mode, such as memory paging, are available.

Certain device drivers and programs can access extended memory, e.g.

Ramdrive.

EXPANDED memory is often called LIM memory because of the EMS

specification developed jointly by Lotus, Intel, and Microsoft. It

describes ways in which programs may acces up to 32Meg of memory

outside conventional memory. Long ago, the only way to acces expanded

memory was to fit a system with a special memory expansion card.

Now, using virtual 8086 mode on 386 and 486 systems, those expansion

cards can be emulated. Expanded memory can also be called paged memory

because it is devided into 16K pages. These pages are accessed using

a block of four 16K pages, called a page frame. A 64K area of upper

memory must be reserved for a page frame if access to expanded memory

is required.

A device driver called the Expanded Memory Manager (EMM) coordinates

access to expanded memory. The EMM driver arranges things so that the

physical pages in the page frame always give access to the correct

logical pages in expanded memory. When data is read from or written to

a physical page in the page frame, EMM ensures that the data is read

from or written to the associated page of memory on the expanded memory

card.

COMMON MEMORY ERRORS:

CMOS MEMORY SIZE MISMATCH: Enter CMOS setup and select Save and Exit.

PARITY ERROR ????: System memory parity error, address undeterminable.

OFF BOARD PARITY ERROR: Parity error in memory on I/O card.

PARITY:

A PARITY BIT is a check bit appended to a unit of data to make the

sum of the total bits even or odd. Computers are designed to use only

one type of parity throughout the system. A PARITY CHECK is performed by

testing a unit of data for odd or even parity to determine whether an

error has occured during reading, writing, or transmission of data. A

parity bit is calculated and compared to the parity bit already appended

to the data. If they are equal, the data is assumed to be okay. If they

are unequal, a PARITY ERROR has occurred and the data must be read again.

UNRECOGNIZED MEMORY:

Some motherboards have configuration jumpers or dipswitches to select

the memory size or type. This information can be obtained from the

motherboard manufacturer. Other boards may have an extended setup in the

BIOS which must be configured for size and type. Information on which

registers must be set and what values they must have can be obtained from

the manufacturer of the chipset which the motherboard uses. AMI mother-

boards have an auto-recognition feature which will sense the amount and

type of memory.

When a board is properly configured and all the memory is still not

recognized, the first thing to try is remove the memory and re-seat it.

If this does not help, try new known-good memory. When setup is proper

and memory is known-good, the problem lies in the motherboard itself.

Some traces could be broken, sockets could have cold solder joints, or

some other physical problem. Consult the dealer or the manufacturer if

possible for instructions on repair procedure.