Notes about the sound drivers

Note: "Mix data not allocated" messages mean that you do NOT have sufficient

(conventional) memory to run the driver.

Be sure to check the driver screen (Shift-F5) of your soundcard!

��������������������������������������������������������������������������������

Driver Summary

Driver                  Def/Max Chan.   Stereo  Bits    Mixing Rate/Resolution

GUS, Hardwave           32/32           Yes     16      19->44kHz (A)

GUS, Software *         64/256          Yes     16      19->44kHz

Interwave, Hardware     32/32           Yes     16      44kHz

Interwave, Software *   64/256          Yes     16

Sound Blaster 1.0       64/256          No      8       12kHz->22kHz

Sound Blaster 2.0       64/256          No      8       12kHz->44kHz

Sound Blaster Pro       64/256          Yes     8       6kHz->22kHz (B)

No      8       12kHz->44kHz (B)

Sound Blaster 16        64/256          Yes     16      12kHz->44kHz

Sound Blaster AWE 32    30/30           Yes     16      44kHz

Pro Audio Spectrum      64/256          Yes     8       12->44kHz

Pro Audio Spectrum 16   64/256          Yes     16      12->44kHz

PC Speaker              64/256          No      5-7     12->44kHz

DAC on LPT              64/256          No      8       12->44kHz

Disk Writer             256/256         Yes     16      22->64kHz

Notes

A) Depends on number of channels used. The hiquality GUS driver reinitialises

the GUS continually to use as few channels as necessary.

B) The mixing rate of the SBPro depends on whether playback is stereo or mono

Drivers with * next to them have NOT been completed...

��������������������������������������������������������������������������������

Gravis UltraSound, Hardware mixing (ITGUS.DRV)

This file actually contains two drivers in one file. The first is

accessed just by using IT (with no command line parameters, or with

/s4 for Gravis UltraSound). This is equivalent to the original

internal driver that came with previous versions of Impulse Tracker.

The second driver is selected by providing the correct IRQ for the

GF1 chip. (The second-to-last number of your ULTRASND environment

variable). This is an IRQ driven routine, which means that it'll

work in the background of Windows '95. But note that the timing for

this is NOT as accurate as the timing in the first driver. There is

also a possibility that multitasking OSs can sometimes (although

rarely) cause some settings to the GUS to be missed (which will cause

a note to play unexpectedly). This can be fixed just by restarting

playback. There is NO check for the correctness of the IRQ provided.

The Gravis UltraSound *CANNOT* cope with 16-bit samples greater than

256k-bytes. This is equivalent to 128k-length samples. Also, 16-bit

samples cannot cross 256k boundaries on the GUS, meaning that the

amount of memory you have on the card may decrease by more than you

expect when you load a 16-bit sample.

You cannot choose the mixing rate for the GUS - the mixing rate is

dependent on the number of channels playing. This driver continuously

reinitialises the GUS to use as few channels as possible. You can

further restrict the number of channels used with /Lxx on the command

line of Impulse Tracker.

Completed.

Gravis UltraSound 2, Hardware mixing (ITGUS2.DRV)

If the first Gravis UltraSound driver clicks continuously when nothing

is supposed to be playing, use this driver ("IT /sITGUS2.DRV").

This driver file also contains two drivers - check above on how to

access the second driver.

Completed.

Gravis UltraSound Lo-freq, Hardware mixing (ITGUSLO.DRV)

Only use this driver if notes do *NOT* finish playing off correctly

on your GUS. ("IT /sITGUSLO.DRV" or copy ITGUSLO.DRV over ITGUS.DRV)

Gravis UltraSound Mixed Driver

Not finished.

��������������������������������������������������������������������������������

InterWave Driver - Hardware mixing (ITIW.DRV)

(This includes GUS PnP, GUS PnP Pro, WavExtreme 32 Pro + more)

You *NEED* to have RAM onboard to use this driver, otherwise your

Interwave card will NOT be detected.

This file actually contains two drivers in one file. The first is

accessed just by using IT (with no command line parameters, or with

/s6 for AMD Interwave IC). This is similar to the original internal

GUS driver that came with previous versions of Impulse Tracker.

The second driver is selected by providing the correct IRQ for the

Interwave chip. (This is the value given in Windows'95/settings/

control panel/system/Interwave SYNTH/IRQ). This is an IRQ driven

routine, which means that it'll work in the background of Windows '95.

But note that the timing for this is NOT as accurate as the timing in

the first driver. There is also a possibility that multitasking OSs

can sometimes (although rarely) cause some settings to the GUS to be

missed (which will cause a note to play unexpectedly). This can be

fixed just by restarting playback (or reinitialising in severe cases).

There is NO check for the correctness of the IRQ provided.

The Interwave driver contains handlers for two different memory modes

on the Interwave - the more memory efficient mode is where the amount

of ram is directly compatible with the interwave, the second is where

the DRAM configuration is NOT directly compatible with the interwave

and the driver has to handle the RAM slightly more explicitly, which

causes the loss of memory-usage efficiency.

Here are the modes directly compatible with the interwave:

Bank 0         Bank 1          Bank 2          Bank 3          Total

256Kb          0               0               0              256Kb

256Kb          256Kb           0               0              512Kb

256Kb          256Kb           256Kb           256Kb           1MB

256Kb          1MB             0               0             1.25MB

256Kb          1MB             1MB             1MB           3.25MB

256Kb          256Kb           1MB             0              1.5MB

256Kb          256Kb           1MB             1MB            2.5MB

1MB            0               0               0               1MB

1MB            1MB             0               0               2MB

1MB            1MB             1MB             1MB             4MB

4MB            0               0               0               4MB

* 4MB            4MB             0               0               8MB

* 4MB            4MB             4MB             4MB            16MB

* These modes cannot be handled by the first driver, so are actually

handled in the second mode.

The mixing rate for the Interwave driver is fixed at 44100Hz

(CD quality)

Bug warning: If the sound does NOT play properly, you may need to

run IWINIT before running Impulse Tracker

Completed.

Interwave Mixed driver

(This includes GUS PnP, GUS PnP Pro, WavExtreme 32, WavExtreme 32 Pro + more)

Not finished.

��������������������������������������������������������������������������������

Sound Blaster 1.0 driver (ITSB.DRV)

The Sound Blaster has a mixing range of 12000->21739 Hz. You CANNOT

hear any stereo (or surround) effects with this driver, because the

Sound Blaster does NOT support stereo.

Hardware detection routines are used for Address (eg. 220h),

environment is checked for IRQ and DMA.

Command line arguments for IRQ and DMA are NOT checked but assumed

correct IF this driver is explicitly selected

(IT /s1 or IT /sITSB.DRV).

Completed.

Sound Blaster 2.0 driver (ITSB2.DRV)

The Sound Blaster 2 driver is basically the same as the Pro driver

with stereo options removed... (ie SB2 cannot do stereo). The

mixing range is from 12000 to 43478 Hz.

Hardware detection routines are used for Address (eg. 220h),

environment is checked for IRQ and DMA.

Command line arguments for IRQ and DMA are NOT checked but assumed

correct IF this driver is explicitly selected (IT /sITSB2.DRV).

Completed.

Sound Blaster Pro driver (ITSBPRO.DRV)

The Sound Blaster Pro has a mixing range of 12000->43478 in mono mode,

or 6000->21739 in stereo mode.

Hardware detection routines are used for Address (eg. 220h),

environment is checked for IRQ and DMA.

Command line arguments for IRQ and DMA are NOT checked but assumed

correct IF this driver is explicitly selected

(IT /s2 or IT /sITSBPRO.DRV).

Completed.

Sound Blaster 16 driver (ITSB16.DRV)

The Sound Blaster 16 has a mixing range of 12000->45454 in either mono

or stereo modes.

If you specify this driver ( IT /s3 or IT /sITSB16.DRV ) AND an IRQ

or DMA, IT will try to *FORCE* the SB16 to use this IRQ/DMA...

eg. On my system, I have my SB16 configured to IRQ 2, DMA 5, but I

can force it to use IRQ 7, DMA 0 with IT /s3 /i7 /d0

Hardware detection routines are used for all Address, IRQ and DMA.

Note: If you select either of the 32-bit mixing modes, then volumes

between 0->32768 are used internally instead of 0->128.

Completed.

��������������������������������������������������������������������������������

Sound Blaster AWE 32 driver (ITAWE32.DRV)

The Sound Blaster AWE 32 driver directly uses the EMU8000 synth chip.

This synth chip has several limitations which you should be aware of:

� It can only use 16-bit samples.

8 bit samples are automatically converted by IT (so that's not a

problem), but your free memory may decrease by double of what you

expect. (eg. you will need at least 1MB of memory to load 512k

of 8 bit samples)

� It doesn't support ping pong loops or no loops.

IT will automatically expand ping pong loops and will pad non-looped

samples with silence, but this makes sample sustain loops impossible

to implement fully. It also means that ping pong looped samples

could take up to double the memory of forwards looped samples.

Sustain loops will NOT operate on the AWE32 driver.

If you change the loop type from none->forwards or

forwards->ping pong or ping pong->none, you WILL need to reload

the samples each time (Ctrl-G). If you change the loop points on

a sample, you will have to reload the samples (Ctrl-G)

� Has a limited frequency range - from the programming information,

it seems that it is impossible to play a note at above 176kHz.

This equates to any notes 2 octaves above middle C (or higher) for

a sample at 44kHz. This should not cause problems for *MOST* people,

but if you have some notes that won't play, this may be the reason.

Note: The Address used for the SB AWE 32 is the address of the EMU8000,

NOT the address of your SB. (eg. for command line params)

Note: This driver is NOT used as a default, as many users would benefit

more from the SB16 driver. (I recommend having at least 2MB

of memory if you want to use this driver). IT /s7 to use this

if you DO have an AWE32

Note: This driver can operate in Win95. In this mode, it uses a

different mechanism which allows the playing of music in the

background, but timing is *NOT* as accurate here (accurate to

around 100 milli seconds as opposed to 800 nano seconds per

tick)

Completed.

��������������������������������������������������������������������������������

PC Speaker (ITPCSPKR.DRV)

Nothing much else to say here, except... GET A SOUND CARD! :)

Note: On the info page, using the 'variables' display WILL distort

PC Speaker output. Also, it has been found that the Info Page

screens and the Pattern Editor cause a noticeably higher amount

of hiss through the speaker.

Note: This driver will *NOT* necessarily work on laptop's

piezo-electric speakers.

Note: No driver screen available.

Completed.

��������������������������������������������������������������������������������

Pro Audio Spectrum (ITPAS.DRV)

Pro Audio Spectrum 16 (ITPAS16.DRV)

BIG thanks to Pelusa for VITAL programming information for this!!

BIG thanks to MZ/PoP for lending me a PAS16 to stuff around with!!

Note: These drivers will *NOT* work in the background of Win95,

although they will work fine in the foreground.

Note: You NEED to have the MVSOUND.SYS driver installed for these

to operate.

Completed.

��������������������������������������������������������������������������������

DAC on LPT 1/2 Drivers

These drivers are almost exactly the same as the PC speaker drivers,

with only minor modifications.

To use these, run IT /sITLPT1.DRV or IT /sITLPT2.DRV - depending on

which LPT you have your DAC plugged into.

Note: No driver screen available.

Completed.

��������������������������������������������������������������������������������

Disk Writer

This RAW file writer is NOT available for public distribution.

Contact me if you wish to obtain this - it will NOT be made

available without some sort of (monetary) agreement.

Details

� 16 bit Stereo/Mono output

� 22kHz to 64kHz output frequency

� 16 bit interpolation (65536x 'oversampling')

� 32 bit mixing

� Software volume ramping (set at 500th of a second or less)

The disk writer also uses volumes ranging between 0 and 32768

instead of the standard 0 to 128.

Completed.

��������������������������������������������������������������������������������