DYNAVIEW.DOC                MATROX GRAPHICS INC.          April 1, 1996

Dynaview Advanced Features

This file describes the advanced features of the MATROX MGA DynaView

AutoCAD driver for DOS. The following topics are covered :

1.  Customizing the AutoCAD MNU file

2.  Making DynaView menus externally

3.  DEF_MENU

4.  DEF_BUTTON

5.  Decompiling a menu

6.  Customizing DynaView pop-up menus

7.  Using multiple configurations

8.  Customizing the color palette (256 color modes only)

9.  Blackvec utility

10. Matrox AutoLISP Commands

1. Customizing the AutoCAD MNU file

===================================

Adding to a Side-bar or Pull-down menu

Matrox DynaView commands are primarily accessible from the keyboard

and through the DynaView menus.

These commands can also be made available from the AutoCAD side menu

or pull-down menus. If you wish to add the DynaView commands to your

AutoCAD menus, you can use the acad.mnu and matrox.mnu files which

are copied to the driver directory during installation.

First time customization

If you are using AutoCAD Release 12 or 13, and have never customized your

AutoCAD menu file before, you can simply copy the proper Matrox acad??.mnu

file to your AutoCAD directory. This special .mnu files contain pull-down

and side-bar menus for DynaView/2D and 3D.

Note: This menu file assumes that the DynaView/3D ADS application is

located in the default directory (\MGA\ACADRV). If you installed to a

different directory, edit the acad.mnu file, search for MGA (there

are four occurrences) and correct the path statement to point to your

installation directory.

Already customized

If you have previously customized your AutoCAD menu, or are using

AutoCAD Release 11, you can use the add-on.mnu file, which contains

statements to make pull-down and side bar menus for the DynaView

commands. In this case, you would copy the lines for the menu you

want, into your acad.mnu file. We assume in this case that you are

familiar with the AutoCAD menu file structure. If not, refer to your

AutoCAD documentation.

Note: If you change the command string called by the DynaView

commands in the configuration facility, you must also update your

acad.mnu file to reflect these changes. Otherwise, the pull-down and

side bar menus will not work for those commands you have changed.

2. Making DynaView menus externally

===================================

MENU_CMP compiler

DynaView menus created externally are a little more complicated

to make, but you have more flexibility in the layout, compared to

when you use the MCREATE command.

To make an external menu, you need a text editor

and the menu_cmp menu compiler. The menu description file must

have the .def extension, from which menu_cmp will create

an associated .bin file, identical to the .bin files

you can create within AutoCAD with MCREATE.

Once a .bin file is created, the menu can be loaded while you are

running AutoCAD. If you use the NO_LOCK switch (discussed below), the

menu buttons can be edited with the Shift+Click feature.

A menu description file contains two statement types:

- One DEF_MENU statement, which specifies the total menu size,

the initial position switch, and the string to display in the title

bar.

- Many DEF_BUTTON statements, which provide the necessary

information for each button contained in the menu: button position

and size, command string to call, text label to display, slide to

display,switch to select text label or slide.

3. DEF_MENU

===========

The syntax of the DEF_MENU statement is:

DEF_MENU(x_size, y_size, CURSOR_POS or DEFAULT_POS, "title", LOCK or No_LOCK)

Parameters

x_size, y_size

The x_size and y_size are in character units. Don't forget that

characters are typically twice as high as they are wide, so to get a

square menu, you must make the x_size equal to twice the y_size.

CURSOR_POS or DEFAULT_POS

The CURSOR_POS switch indicates that the menu should be presented at

the cursor position. The DEFAULT_POS switch indicates that the menu

should be presented at the same position as last time. The very first

time a menu is called, DEFAULT_POS is the same as the cursor

position, but on subsequent requests it will be at the last position

you left the menu. Either CURSOR_POS or DEFAULT_POS must be specified.

title

The title must fit in the title bar, otherwise it will be clipped.

Remember that the title bar also contains two icons (one to pin the

window, and one to put it in the background), that use the equivalent

of four characters.

LOCK

When the LOCK switch is used, the menu cannot be edited inside

AutoCAD with the Shift+click method. The default switch is NO_LOCK.

4. DEF_BUTTON

=============

The syntax of the DEF_BUTTON statement is:

DEF_BUTTON(x_pos, y_pos, x_size, y_size, "command_string",

"text_label", "slide_name" or "slide_lib(slide_name)", DISPLAY_TEXT

or DISPLAY_ICON)

Parameters

position and size

The position and size parameters are in character units, but you can

use fraction values. It is however recommended to limit yourself to

using .5 and .25, otherwise rounding could result in menus that are

not very attractive.

command string

The command string can be up to 80 characters and it supports the

special characters (`;', `^C', `^O', etc.).

text_label

The text label should be no longer than the button's x_size,

otherwise it will be clipped. To be aesthetic, the text label should

be one character shorter than the button width, otherwise the string

will overwrite the button shadows. If the text label is smaller than

the button width, it will be centered. The text label can be an empty

string (` '). In this case, nothing will be displayed in the button,

but the button will still be functional. Note that it is not possible

to have a two-line text label. If you absolutely need this, use a

slide.

Slide_name

The slide can either be a separate slide file or a slide taken from a

slide library. In both cases, do not include the file name extension.

The slide name can be an empty string (''). In this case, nothing

will be displayed in the button, but the button will still be

functional.

DISPLAY_TEXT or DISPLAY_ICON

The DISPLAY_TEXT switch indicates that the provided text label should

be displayed. The DISPLAY_ICON indicates that the provided slide icon

should be displayed. Either DISPLAY_TEST or DISPLAY_ ICON must be

specified.

Comments

Aside from the two statements described above, blank lines and

comments can be put in the .def file. Comments must start with \* and

terminate with *\<|>. They may span more than one line.

Example

DEF_MENU( 15,14,DEFAULT_POS,"Draw Menu" )

DEF_BUTTON(1.0,1.0,6,3,"^C^CLINE","line","matrox(line)",DISPLAY_ICON)

DEF_BUTTON(7.5,1.0,6,3,"^C^CPLINE","pline","matrox(pline)",DISPLAY_ICON)

DEF_BUTTON(1.0,4.0,6,3,"^C^CARC","arc","matrox(arc)",DISPLAY_ICON)

DEF_BUTTON(7.5,4.0,6,3,"^C^CCIRCLE","cir","matrox(circle)",DISPLAY_ICON)

DEF_BUTTON(1.0,7.0,6,3,"^C^CELLIPSE","ellip","matrox(ellipse)",DISPLAY_ICON)

DEF_BUTTON(7.5,7.0,6,3,"^C^CPOLYGON","polyg","matrox(polygon)",DISPLAY_ICON)

DEF_BUTTON(1.0,10.0,6,3,"^C^CPOINT","point","matrox(point)",DISPLAY_ICON)

DEF_BUTTON(7.5,10.0,6,3,"^C^CDONUT","donut","matrox(donut)",DISPLAY_ICON)

5. Decompiling a menu

=====================

Converting a .bin file to a .def file

The MENU_CMP utility features a -d switch, which lets you decompile a

.bin file into a .def file. This feature is useful to get an ASCII

copy of:

- a menu created inside AutoCAD with MCREATE.

- an external menu that you have modified with the Shift+Click

feature.

When decompiling it is possible to specify a different file name for

the .def file. This is useful to avoid over writing an existing .def

file of the same name.

Example:   menu_cmp -d mymenu mymenu2

The above command line decompiles mymenu.bin into mymenu2.def.

6. Customizing DynaView pop-up menus

====================================

As stated earlier in this manual, the DynaView pop-up menus are

regular DynaView menus. Since we include the .def files (mpopup.def

and mpop3d.def) you can edit the menus inside AutoCAD (with

Shift+Click), or you can also modify the menu layouts

and recompile them. The preceding sections give you all the details.

Note: The mpopup.def file uses the default DynaView command names.

If you have changed any of these command names within the

configuration facility, you must update the mpopup.def file

accordingly, and recompile the menu.

7. Using multiple configurations

================================

Place acadrv.cfg in different directories

It is possible to set up the DynaView driver so that multiple

configurations can be available.

Determining the configuration

The driver uses the acadrv.cfg file to determine its configuration.

When AutoCAD is started, the driver looks first for the acadrv.cfg

file in your current directory, then in the directory specified by

the ACADRV environment variable.

The acadrv.cfg file saves the status of items such as:

- the resolution and font size

- the MCONFIG flags (scroll bars, bird's eye mode, etc.)

- the MBUTTON assignments

- the window arrangement (position, size, pinned windows)

- other parameters from the configuration program

Using a different configuration

Multiple configurations can be accomplished by making and copying

different acadrv.cfg files into different directories, and then

starting AutoCAD from one of those directories.

You can also start up with different configurations by:

- Making multiple configuration files with different names

(in the same acadrv directory).

- Have a batch file which copies the desired configuration

file over acadrv.cfg before you start AutoCAD.

8. Customizing the color palette (256 color modes only)

=======================================================

Custom color palettes are supported by this AutoCAD driver if they

are described in a Matrox palette file. The palette file must be

`activated', by using a special utility program.

Palette File Creation

A palette file is an ASCII file that you create with any text editor.

The file extension must be .pal. In this file, each line contains one

color definition, in the following format: index, red, green, blue.

The file starts with a 16-color palette, followed by a 256-color

palette.

A sample palette file, standard.pal, is included with the

distribution disk. It defines the standard 16 and 256 color palettes

as stated by AutoCAD.

To create a custom palette, you should make a copy of  standard.pal

to use as a starting point. Be careful not to change the file

formatting, or to delete lines.

Palette File Activation

To activate your custom color palette, you must run the palname.exe

utility program, which is included on the distribution diskette. This

utility will update the Matrox configuration file, acadrv.cfg, with

your new palette file name, and from then on your custom palette will

be used. Your palette file must have the extension .pal. An example

of a palette file is the file standard.pal which is shipped with the

MGA AutoCAD driver.

Example :  palname standard.pal

Note: Every time your reset the driver to the factory defaults (in

the configuration facility), the driver will stop using the custom

palette, and you must re-use palname to activate your custom

palette.

Color limitations

Customizing the color palette is a powerful and potentially dangerous

feature! Please note the following points when creating your palette:

--- Color 0 is the background color. If you change it, check that no

other colors in the palette are equal or very close to your new

background color. Those colors would become barely visible, or even

invisible. The normal practice is to set all colors that are close

to the new background color, to black. (This is what the driver does

when you select gray background in the configuration.)

--- Colors 1 to 8 are used in the user interface. If you change one

of them, it will affect not only your drawings but also the user

interface. If, afterwards, you want to adjust the user interface

colors through the configuration facility, the configuration will

still call the colors by their original name (i.e. index no.2 will

be called `yellow'). As a reference for you, see the table below

which shows the values assigned for colors 1 to 8.

Color Index    Name          r,g,b values

-------------+-------------+-------------------

1           red           255,0,0

2           yellow        255,255,0

3           green         0,255,0

4           cyan          0,255,255

5           blue          0,0,255

6           magenta       255,0,255

7           white         255,255,255

8           gray          160,160,160

Animation limitations

When you run in 8-bit mode with 256 colors, this has an impact on

features that use animation. Due to these color limitations, the user

interface colors could change during an animation command, for any

menus that overlap a viewport in animation mode.

Use these values

To make your custom colors, and prevent the user interface colors

from being affected, use only 0 and 255 as intensity values for the

red, green, and blue components. Note that this limitation is not

present when you are running in 16bpp and 24 bpp modes.

9. Blackvec utility

===================

The automatic display list clean-up feature can be disabled through

the blackvec utility. The command line syntax is shown below:

blackvec off (to disable black vector cleanup)

blackvec on (to enable black vector cleanup)

Note: Every time you reset the driver to the factory defaults (in the

configuration utility), the driver will reset black vector clean-up

to ON.

10. Matrox AutoLISP Commands

============================

========================================================================

MATRDINQ

========================================================================

Summary

Inquire if a viewport is in rendering mode.

Usage

( matrdinq [vport] )

Description

If the vport argument is given, matrdinq returns a value for that

viewport. If the vport argument is not given, matrdinq will check

the current viewport to see if it is in rendering mode.

Returns

Value       Meaning

----------+----------------------------

nil         Viewport is not rendered.

non-nil     Viewport is rendered.

========================================================================

MATDISPLAY

========================================================================

Summary

Change the display option used for rendering.

Usage

( matdisplay vport mode )

Arguments

vport    Viewport to be affected

mode     Display option ("W", "H", "B", "F", or "G") to be used

Description

The matdisplay function changes the display option of a rendered

viewport. The mode argument can be "W" (Wireframe),"H" (Hidden),

"B" (Bordered),"F"(Flat), or "G"(Gouraud shaded)

Returns

Value    Meaning

-------+------------------------------------------------------

nil      Indicates an error condition (not a rendered viewport,

invalid argument, etc.).

void     No error condition.

========================================================================

MATVPOINT

========================================================================

Summary

Change or request the viewing parameters of a rendered viewport.

Usage

To inquire: - ( matvpoint  vport "?" )

To change: - ( matvpoint vport x )

Where x = mode (argument value)  (arg. value) ...)

Arguments

Name         Meaning

-----------+--------------------------------

vport        Viewport number

mode         "o": orthogonal

"p": perspective

[?]          Display the viewing parameters.

-------------------------------------+---------------------------------------

In Orthogonal view only:             | In Perspective view only:

-------------------------------------+---------------------------------------

["c" 3dpoint] Set the view center    | ["f" real]       Set the focal length

-------------------------------------+---------------------------------------

["d" 3dpoint] Set viewing direction  | ["ca" 3dpoint]   Set camera location

-------------------------------------+---------------------------------------

["h" 3dpoint] Set the view height    | ["ta" 3dpoint]   Set target location

-------------------------------------+---------------------------------------

| ["tw" real]      Set the twist angle

-------------------------------------+---------------------------------------

========================================================================

MATDBLBUF

========================================================================

Summary

Select double-buffering mode for a rendered viewport.

Usage

( matdblbuf vport display_buf draw_buf )

Arguments

vport         Viewport number

display_buf   Display buffer

draw_buf      Drawing buffer

Both display_buf and draw_buf can assume the following values:

Value       Configuration

---------+----------------------------

0         Full frame buffer

1         Lower half of frame buffer

2         Upper half of frame buffer

Description

The matdblbuf function enables or disables double buffering.

Returns

void

Important Points:

If you cancel an AutoLISP program which uses double-buffering, make

sure to reset the board to `Full frame buffer' as soon as possible,

by typing:

matdblbuf vport 0 0

If this is not done, many redraw type commands (REDRAW, MATVPOINT,

MATDISPLAY) will not work. It will then be necessary to exit

RD mode (MATEXIT, FLUSH, REGEN) and re-render the viewport.

========================================================================

MATLIGHT

========================================================================

Summary

Change or request lighting parameters.

Usage

(matlight "?") to inquire current lighting

(matlight "c" light1 light2 ...) to create lights

(matlight "m" oldlight newlight) to modify a light

(matlight "d" light1 light2 ...) to delete lights

(matlight "u") to update drawing file lighting information

Arguments

Ambient light:

--------------

(list "al" (list "ln" "light_name") (list "di" r g b))

Model Directional lights:

-------------------------

(list "mdl" (list "ln" "light_name")

(list "di" r g b) (list "sp" r g b)

(list "po" x y z) (list "ap" x y z))

User Directional lights:

------------------------

(list "udl" (list "ln" "light_name")

(list "di" r g b) (list "sp" r g b)

(list "ha" angle) (list "ea" angle))

Positional lights:

------------------

(list "pl" (list "ln" "light_name")

(list "di" r g b) (list "sp" r g b)

(list "rl" residual) (list "rd" distance)

(list "po" x y z) )

Spot lights:

------------

(list "sl" (list "ln" "light_name")

(list "di" r g b) (list "sp" r g b)

(list "rl" residual)

(list "ca" angle) (list "cf" angle)

(list "fd" string)

(list "po" x y z) (list "ap" x y z))

Where:

"ln" = light name string (up to 31 characters)

"di" = diffusion color (0.0 <= r, g, b, <= 1.0)

"sp" = specular color (0.0 <= r, g, b, <= 1.0)

"po" = light position (in WCS Coordinate)

"ap" = light aim point (in WCS Coordinates)

"ha" = horizontal angle in degree (-180.0 <= angle <= 180.0)

"ea" = elevation angle in degree (-90.0 <= angle <= 90.0)

"rl" = residual light factor ( 0.0 <= factor <= 100.0 %)

"rd" = residual light distance (in WCS Coordinates)

"ca" = cone aperture angle in degrees (0.0 <= angle <= 160.0)

"cf" = cone fall-off angle in degrees ( ca <= angle <= 160.0)

"fd" = falloff distribution string ("linear", "square", or "cubic")

Returns

Value      Meaning

---------+------------------------------------------------

non-nil    No error

nil        Error (invalid argument type, invalid number of

arguments, no light on layer, light name already

exists, etc.

list       Current light list (with "?" only)

Examples

; create Ambient light source and one Directional light source

(matlight  (list "c" (list "al" (list "ln" "am01")

(list "di" 0.2 0.3 0.4))

(list "dl" (list "ln" "dir02") (list "di" 0.1 0.2 0.3)

(list "po" 12.0 23.0 123.0)

(list "ap" 22.0 33.0 143.0))))

; delete Ambient light source (notice that the data for it must be provided)

(matlight (list "d" (list "al" (list "ln" "light01")

(list "di" 0.2 0.3 0.4))))

; modify Ambient light source

(matlight (list "m" (list "al" (list "ln" "am") (list "di" 0.2 0.3 0.4))

(list "al" (list "ln" "am2") (list "di" 0.3 0.4 0.6))))

Note: All r, g, b parameters are the red, green, and blue components

of a light, and they must be within the range of 0.0 to 1.0.

========================================================================

MATMATERIAL

========================================================================

Summary

Change or request material properties parameters.

Usage

(matmaterial "?" entityname) to inquire

(matmaterial "r" entityname) to reset

(matmaterial "c" entityname [argument_list]) to change

Arguments

["am" r g b]  Ambient color (0.0 <= r g, b, <= 1.0)

["di" r g b]  Diffuse color (0.0 <= r, g, b, <= 1.0)

["em" r g b]  Emission color (0.0 <= r, g, b, <= 1.0)

["sp" r g b]  Specular color (0.0 <= r, g, b, <= 1.0)

["sc" coef ]  Specular coefficient (0.0 <= coefficient <= 1.0)

["ma" "string"] Optional material name (31 char max)

Returns

Value      Meaning

---------+-----------------------------------------------

non-nil    No error

nil        Error (invalid argument type, invalid number of

arguments, no material on entity, etc.)

list       Current material data (with "?" only)

Example

(matmaterial "c" ename (list "di" 0.1 0.1 0.1) (list "am"

0.2 0.2 0.2)  (list "ma" "plastic"))

========================================================================

MATCFG

========================================================================

Summary

Change or request configuration parameters.

Usage

(matcfg "?") to inquire

(matcfg [argument_list] ) to set

Arguments

["l" "light_layer"]   Light source layer

["r" method]          Rendering method ("w" or "g") (now referred

to as `Display Option' in the documentation)

["b" r g b]           Background color

["a" min max]         Min and max arc segmentation

["sp" "y" or "n"]     Spline segmentation

["s" "y" or "n"]      Automatic selection

["f" "y" or "n"]      Full Gouraud flag

["n" level]           Level for 3D face nesting

["t" angle]           Threshold angle, in degrees

["m" "a" or "p"       Default material, absolute or percentage,

followed by one or all of the parameters

below:

("am" r g b)         ambient color (0.0 <=  r, g, b, <= 1.0)

("di" r g b)         diffuse color (0.0 <=  r, g, b, <= 1.0)

("em" r g b)         emission color (0.0 <=  r, g, b, <= 1.0)

("sp" r g b)         specular color (0.0 <=  r, g, b, <= 1.0)

("sc" coef )]        specular coefficient (0.0 <= coefficient <= 1.0)

Returns

Value       Meaning

----------+------------------------------------------------

list        Current parameters (with "?" only)

non-nil     No error

nil         Error (invalid argument type, invalid number of

arguments, etc.)

Examples

; set display option to Gouraud, and nesting level to 2

(matcfg (list (list "r" "g") (list "n" 2) ) )

; set default material colors to Absolute, with no Ambient reflectivity

(matcfg (list (list "m" "a" (list "am" 0.0 0.0 0.0 ) ) )

Note: All r,g,b parameters are the red, green, and blue components of

a light, and they must be within the range of 0.0 to 1.0.